D5S

Ducati Hypermotard 796/1100 2010-

Mounting kit for the left:		     		Mounting kit for the right:

| ID | Description | Cat.no. | pcs |
| --- | --- | --- | --- |
| 1 | Moto adapter | 302 | 1 |
| 2 | Bolt M6x25 | 877 | 3 |
| 3 | Bolt M4x16 | 876 | 2 |
| 4 | Steel nut M12 | X12 | 1 |
| 5 | Stainless steel rod M12x365x1,25mm | N1 | 1 |

| ID | Description | Cat.no. | pcs |
| --- | --- | --- | --- |
| 1 | Moto adapter | 302 | 1 |
| 2 | Bolt M6x25 | 877 | 3 |
| 3 | Bolt M4x16 | 876 | 2 |
| 4 | Steel nut M12 | X12 | 1 |

Use Stainless steel rod (ID5) for connecting both sides.

Installation:

- Remove original bolt M12 from the frame of motorcycle.

- Insert stainless steel rod (ID5) into the frame of motorcycle.

- Tighten both adapters using nut (ID4).

- Fit slider and tighten it using bolts (ID2).

- Insert coloured cover into slider and tighten it using two botls (ID3).

We recommend to use the glue on all threads (medium strong – blue colour).

4

4

1

1

5

2

3

3

2

Streetfighter 1098