D5

Ducati Hypermotard 1100 2007-

Hypermoterd 796/ Streetfighter 1098

| Kit for the left side |  |  |  |  | Kit for the right side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Description | Cat.no | Pcs |  |  | No. | Description | Cat.no | Pcs |  |
| 1 | Steel nut M12 | X12/X2 | 1 |  |  | 1 | Steel nut M12 | X12/X2 | 1 |  |
| 2 | Duralumin cylinder d=70mm | 56 | 1 |  |  | 2 | Duralumin cylinder d=70mm | 56 | 1 |  |
|  | Stainless steel bar 12x(1,25)x405x30x30 |  | 1 |  |  |  |  |  |  |  |

Installation:

- dismount the long original bolt M12

- insert through bar the stainless steel bar M12x405x1,25mm

- give duralumin cylinder (No. 2) on this stainless steel bar both sides

- finally screw up all with nuts M12 (No. 1)

Warning – we recommend to use safety glue on threads of all bolts (moderate strong – blue colour).