D6SDM

Ducati Streetfighter 848 2012-

Kit for left side: 							Kit for right side:

| ID. | Description | Cat.N | Qt. |
| --- | --- | --- | --- |
| 1 | Alu. bracket for SLDM | 537/B | 1 |
| 2 | Alu. spacer d=65 | 41 | 1 |
| 3 | Alu. cone spacer | 149 | 1 |
| 4 | Safety nut M12 | 811 | 1 |
| 5 | Bolt M6x20 | 1024 | 2 |
| 6 | Washer M6 | 1023 | 2 |
| 7 | Steel rod 12x420x30x30 |  | 1 |

| ID. | Description | Cat.N | Qt. |
| --- | --- | --- | --- |
| 1 | Alu. bracket for SLDM | 537/B | 1 |
| 2 | Alu. spacer d=65 | 41 | 1 |
| 3 | Alu. cone spacer | 149 | 1 |
| 4 | Safety nut M12 | 811 | 1 |
| 5 | Bolt M6x20 | 1024 | 2 |
| 6 | Washer M6 | 1023 | 2 |

6

3

4

1

2

3

2

1

6

5

4

7

5

Assembly instruction:

- Dismantle original bolt M12 from frame.

- Place steel rod ID.7 throught the engine, replace orignal bolt.

- Place alu. adapters ID.2 insert on the steel rod from both sides. Smaller diameter into the frame.

- Put cone alu. spacers ID.3 from left and right side on the steel rod.

- Place alu. bracket ID.1 on the steel rod.

- This kit tighten by nuts ID.4.

- Place sliders and fixed it by bolts and washers ID.5 and ID.6.

Warning - on the threads of the screws we recommend using premium adhesive moderate (blue).