D6S

Ducati Streetfighter /S (1098) 2009-

Ducati Streetfighter 848 2012-

| Kit for the left side |  |  |  |  | Kit for the right side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Description | Cat.no. | Pcs |  |  | No. | Description | Cat.no. | Pcs |  |
| 1 | Slider RDmoto | SL | 1 |  |  | 1 | Slider RDmoto | SL | 1 |  |
| 2 | Cover | SLcap.. | 1 |  |  | 2 | Cover | SLcap.. | 1 |  |
| 3 | Adapter moto | DV-302 | 1 |  |  | 3 | Adapter moto | DV-302 | 1 |  |
| 4 | Bolt M6x25 | 877 | 3 |  |  | 4 | Bolt M6x25 | 877 | 3 |  |
| 5 | Bolt M4x16 | 876 | 2 |  |  | 5 | Bolt M4x16 | 876 | 2 |  |
| 6 | Steel nut M12 | X12 | 1 |  |  | 6 | Steel nut M12 | X12 | 1 |  |
|  | Stainless steel bar 12(1,25)x365x30x30 |  | 1 |  |  |  |  |  |  |  |

For connecting of the left and right side use the stainless steel bar (N2) with a length of 365 mm, at both ends of the threads M12x1,25mm, which is included mounting kit.

Installation:

- remove the original bolt

- insert through bar the stainless steel bar (N2)

- give adapter (No. 3) on this stainless steel bar both sides

- use the special nuts (No. 6) for screwing up, you have to mount adapter with thread M6 up, then slider have the right rotation

- put slider (No. 1) and use the bolts (No. 4) for mounting

- insert in slider the cover (No. 2) and use the bolts (No. 5) for gripping

Warning – we recommend to use safety glue on threads of all bolts (moderate strong – blue colour).