D8SDM

Ducati Monster 821/1200 ´14

Left side kit:                           	      Right side kit:

| ID | Description | Cat.no | Pcs |
| --- | --- | --- | --- |
| 1 | Frame | 520 | 1 |
| 2 | Cylinder 80,5mm (front) | 1177 | 1 |
| 3 | Cylinder 75 mm (rear) | 1178 | 1 |
| 4 | Bolt M12x150x1,25 (front) | 984 | 1 |
| 5 | Bolt M12x140x1,25 (rear) | 995 | 1 |
| 6 | Washer M12 | 803 | 1 |
| 7 | Bolt M10x40 | 854 | 1 |
| 8 | Dural. adapter | 10 | 1 |
| 9 | Dural. adapter for SLDM | 537/B | 1 |
| 10 | Bolt M6x20 | 1024 | 2 |
| 11 | Washer M6 | 1023 | 2 |
| 12 | Plastic cover M12 | X60 | 2 |

| ID | Description | Cat.no. | Pcs |
| --- | --- | --- | --- |
| 1 | Frame | 531 | 1 |
| 2 | Cylinder 47,5mm (front) | 1179 | 1 |
| 3 | Cylinder 82 mm (rear) | 1180 | 1 |
| 4 | Bolt M12x120x1,25 (front) | 913 | 1 |
| 5 | Bolt M12x140x1,25 (rear) | 995 | 1 |
| 6 | Washer M12 | 803 | 1 |
| 7 | Bolt M10x10 | 854 | 1 |
| 8 | Dural. adapter | 10 | 1 |
| 9 | Dural. adapter SLDM | 537/B | 1 |
| 10 | Bolt M6x20 | 1024 | 2 |
| 11 | Washer M6 | 1023 | 2 |
| 12 | Plastic cover M12 | X60 | 2 |

Left side

3

11

picture.1

1

2

4

9

5

8

10

7

11

6

1

3

Right side

2

9

4

8

6

5

8

10

7

11

- Remove original bolts from the frame of motorcycle.

- Insert dural cylinders (ID2 + ID3) into the frame of motorcycle.

- Mount frame ID1 on the frame of motorcycle using bolts ID4 and ID5+ID6.

- Place plastic covers on bolts.

- Put dural adapter ID9 and SLD adapter ID8 on bot ID7. Mount this kit into the frame.

- Mount slider using two M8 bolts + washers.

We recommend to use the glue on all threads (medium strong).