D8S

Ducati Monster 821/1200 ´14

Left side kit:	                                 Right side kit:

| ID | Description | Cat.no | Pcs |
| --- | --- | --- | --- |
| 1 | Frame | 520 | 1 |
| 2 | Cylinder 80,5mm (front) | 1177 | 1 |
| 3 | Cylinder 75 mm (rear) | 1178 | 1 |
| 4 | Bolt M12x150x1,25 (front) | 984 | 1 |
| 5 | Bolt M12x140x1,25 (rear) | 995 | 1 |
| 6 | Washer M12 | 803 | 1 |
| 7 | Bolt M10x30 | 882 | 1 |
| 8 | Dural. adapter for SLD | 292 | 1 |
| 9 | Bolt M6x25 | 877 | 3 |
| 10 | Bolt M4x16 | 876 | 2 |
| 11 | Plastic cover M12 | X60 | 2 |
| 12 |  |  |  |

| ID | Description | Cat.no. | Pcs |
| --- | --- | --- | --- |
| 1 | Frame | 531 | 1 |
| 2 | Cylinder 47,5mm přední | 1179 | 1 |
| 3 | Cylinder 82 mm zadní | 1180 | 1 |
| 4 | Bolt M12x120x1,25 přední | 913 | 1 |
| 5 | Bolt M12x140x1,25 zadní | 995 | 1 |
| 6 | Washer M12 | 803 | 1 |
| 7 | Bolt M10x30 | 882 | 1 |
| 8 | Dural. adapter for SLD | 292 | 1 |
| 9 | Bolt M6x25 | 877 | 3 |
| 10 | Bolt M4x16 | 876 | 2 |
| 11 | Plastic cover M12 | X60 | 2 |
| 12 |  |  |  |

7

8

Left side

3

1

2

10

9

5

4

11

10

6

1

Right side

7

2

3

10

5

4

6

8

9

12

11

- Remove original bolts from the frame of motorcycle.

- Insert dural cylinders (ID2 + ID3) into the frame of motorcycle.

- Mount frame ID1 on the frame of motorcycle using bolts ID4 and ID5+ID6.

- Place plastic covers on bolts.

- Put dural adapter ID8 on the bolt ID7. Mount this kit on the frame.

- Fit slider using bolts ID9.

- Fit cover using bolts ID10.

We recommend to use the glue on all threads (medium strong).