D8

Ducati Monster 821/1200 ´14

Left side mounting kit:                                 Right side mounting kit:

| ID | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Steel bar protector holder | 520 | 1 |
| 2 | Spacer 80,5mm front | 1177 | 1 |
| 3 | Spacer 75 mm rear | 1178 | 1 |
| 4 | Bolt M12x150x1,25 front | 984 | 1 |
| 5 | Bolt M12x140x1,25 rear | 995 | 1 |
| 6 | Washer M12 | 803 | 1 |
| 7 | Steel box | X1 | 1 |
| 8 | Bolt M10x50 | 849 | 1 |
| 9 | Plastic cover M12 | X60 | 2 |

Left side

3

Obr.1

| ID | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Steel bar protector holder | 531 | 1 |
| 2 | Spacer 47,5mm front | 1179 | 1 |
| 3 | Spacer 82 mm rear | 1180 | 1 |
| 4 | Bolt M12x120x1,25 front | 913 | 1 |
| 5 | Bolt M12x140x1,25 rear | 995 | 1 |
| 6 | Washer M12 | 803 | 1 |
| 7 | Steel box | X1 | 1 |
| 8 | Bolt M10x50 | 849 | 1 |
| 9 | Plastic cover M12 | X60 | 2 |

2

8

7

1

5

4

9

9

6

Right side

3

1

2

7

4

8

5

6

9

9

- Assembly place as you can see on the pictures.

- Dismantle original bolts M12 from left side.

- Place  front and rear spacers into the frame. On the frame mounte steel holder for protector. Put washer on the rear bolt ID.5+ID.6.

- Place plastic cover into the bolts.

- Insert steel box ID.7 into protector, take bolt ID.8 and mount protector.

- Dismantle original bolts M12 from right side.

- Place  front and rear spacers into the frame. On the frame mounte steel holder for protector. Put washer on the rear bolt ID.5+ID.6.

- Place plastic cover into the bolts.

- Insert steel box ID.7 into protector, take bolt ID.8 and mount protector

Warning – use a glue of all threads during assembly. Recommend medium strong (blue).