H10

Honda CBR600RR 2003-2006

| Kit for left side: |  |  |  |  | Kit for right side: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID. | Description | Cat.N | Qt |  |  | ID. | Description | Cat.N | Qt |  |
| 1 | Steel box M10 | X1 | 1 |  |  | 1 | Steel box M10 | X1 | 1 |  |
| 2 | Bolt M12x85x1,25mm | 902 | 1 |  |  | 2 | Bolt M12x85x1,25mm | 902 | 1 |  |
| 3 | Dural.cylinder d=40mm | 4 | 1 |  |  | 3 | Dural.cylinder d=40mm | 4 | 1 |  |
| 4 | Bolt M10x30mm | 882 | 1 |  |  | 4 | Bolt M10x30mm | 882 | 1 |  |
| 5 | Bolt M10x80mm | 852 | 1 |  |  | 5 | Bolt M10x80mm | 852 | 1 |  |
| 6 | Steel adapter | 510 | 1 |  |  | 6 | Steel adapter | 511 | 1 |  |
| 7 | Dural. cylinder d=16mm | 127 | 1 |  |  | 7 | Dural.cylinder d=11mm | 17 | 1 |  |
| 8 | Tape 3M |  | 1 |  |  | 8 | Tape 3M |  | 1 |  |
| 9 | Duralumin rod AL16 (M10)x392x30x30 |  | 1 |  |  |  |  |  |  |  |

This fitting kit is without modification of fairings.

Instruction to assembly:

- Take off left and right hand side fairng.

- Take off original bolt M12 from frame.

- 3M tape stick on the frame on the place wher moto frame is touching with steel adapter ID.6.

- Mounting steel adapters on the motorcycle ID.6, Right hand side adapter insert between the engine and frame. Don´t forget put dural. cylinder ID.7 beteen the adapter and frame.For  thighten use bolt ID.2.

- For thighten on the left side use bolt ID.2. Don´t forget put dural. cylinder ID.7 beteen the adapter and frame.

- Put dural. rod ID.9 between the left and right adapters,this rod mounting by bolts ID.4

- Check all bolts and return back fairings on the bike.

- Into the protectors insert steel boxes ID.1 with bolts ID.5, under the protectors put reduction cylinders ID.3.

- This kit gripping on the steel adapters.

Warning: We recommend use the glue of all threads. Medium strong.

3

1

5

6

2

7

8

2

7

6

3

4

4

5

1

9

Ilustration photo- newer CBR model