H11

Honda CBR900 -´99

Fitting kit left side:		     		Fitting kit right side:

| ID | Description | Cat.N | Qty |
| --- | --- | --- | --- |
| 1 | Steel box M10 | X1 | 1 |
| 2 | Bolt M10X90 | 880 | 1 |
| 3 | Dural.cylinder d=40 | 4 | 1 |
| 4 | Steel adapter | 518 | 1 |
| 5 | Bolt M10x60x1,25 | 896 | 1 |
| 6 | Washer M10 | 801 | 1 |
| 7 | Dural.cylinder d=18 | 1135 | 1 |
| 8 | 3M tape |  | 1 |
| 9 | AL bar AL16x392x30x30 |  | 1 |

| ID | Description | Cat.N | Qty |
| --- | --- | --- | --- |
| 1 | Steel box M10 | X1 | 1 |
| 2 | Bolt M10X90 | 880 | 1 |
| 3 | Dural.cylinder d=40 | 4 | 1 |
| 4 | Steel adapter | 517 | 1 |
| 5 | Bolt M10x60x1,25 | 896 | 1 |
| 6 | Washer M10 | 801 | 1 |
| 7 | Dural.cylinder d=8 | 1134 | 1 |
| 8 | 3M tape |  | 1 |

3

1

1

3

2

2

4

8

9

8

4

5

7

5

7

6

6

Instruction to assembly left side:

- Take out fairing.

- Dismantle bolt M10 from frame.

- Sticking 3M tape ID.4 on the steel adapter, where it is touching with frame.

- Insert dural.cylinder ID.7 into the frame. Take bolt ID.5 and washer ID.6 and mount steel adapter on frame.

- Through the bike insert AL rod ID.9.

- Into the protector insert steel box ID.1 and bolt ID.2. Under the protector insert dural.cylinder ID.3.

- This set grip into AL. rod.

Instruction to assembly right side:

- Take out fairing.

- Dismantle bolt M10 from frame.

- Sticking 3M tape ID.4 on the steel adapter, where it is touching with frame.

- Insert dural.cylinder ID.7 into the frame. Take bolt ID.5 and washer ID.6 and mount steel adapter on frame.

- Into the protector insert steel box ID.1 and bolt ID.2. Under the protector insert dural.cylinder ID.3.

- This set grip into AL. rod.

Warning: We recommend use the glue of all threads. Medium strong.