H12S

Honda CBR929/954RR 2000-2003

| Kit for the left side |  |  |  |  | Kit for the right side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID. | Description | Cat.no | Pcs |  |  | ID | Description | Cat.no. | Pcs |  |
| 1 | slider RDmoto | SL | 1 |  |  | 1 | slider RDmoto | SL | 1 |  |
| 2 | Cover | SLcap.. | 1 |  |  | 2 | Cover | SLcap.. | 1 |  |
| 3 | Moto adapter | DV-310 | 1 |  |  | 3 | Moto adapter | DV-309 | 1 |  |
| 4 | Bolt M6x25 | 877 | 3 |  |  | 4 | Bolt M6x25 | 877 | 3 |  |
| 5 | Bolt M4x16 | 876 | 2 |  |  | 5 | Bolt M4x16 | 876 | 2 |  |
| 6 | Bolt M10x70x1,25mm | 898 | 1 |  |  | 6 | Bolt M10x70x1,25mm | 898 | 1 |  |

Instructions:

- Remove original bolt from the frame of motorcycle.

- Using bolt M10 (ID6) fit dural adapter (ID3). Set rotation before tightening.

- Place slider (ID1) on the dural adapter and tighten it using bolts (ID4)

- Place cover (ID2) into the slider and tighten it using botls (ID5).

We recommend to use the safety glue on all threads (medium strong).

Illustrative photos