H14SD

Honda CBR1000RR 2004-2005

| LEFT SIDE KIT |  |  |  |  | RIGHT SIDE KIT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.no. | Pcs |  |  | ID | Description | Cat.no. | Pcs |  |
| 1 | SLD adapter | 537 | 1 |  |  | 1 | SLD adapter | 537 | 1 |  |
| 2 | Bolt M12x90x1,25mm | 919 | 1 |  |  | 2 | Bolt M12x90x1,25mm | 919 | 1 |  |
| 3 | Dural. cylinder 18mm | 81 | 1 |  |  | 3 | Dural. cylinder 30mm | 82 | 1 |  |
| 4 | Bolt M10x40mm | 854 | 1 |  |  | 4 | Bolt M10x40mm | 854 | 1 |  |
| 5 | Bolt M6x16mm | 962 | 1 |  |  | 5 | Bolt M6x16mm | 962 | 1 |  |
| 6 | Poly nut M10 (11mm) | X7 | 1 |  |  | 6 | Poly nut M10 (11mm) | X7 | 1 |  |
| 7 | Poly nut M6 (11mm) | X8 | 1 |  |  | 7 | Poly nut M6 (11mm) | X8 | 1 |  |
| 8 | Dural adapter | 519 | 1 |  |  | 8 | Dural adapter | 519 | 1 |  |
| 9 | Tape 3M |  | 1 |  |  | 9 | Tape 3M |  | 1 |  |
| 10 | Washer M12 | 803 | 1 |  |  | 10 | Washer M12 | 803 | 1 |  |
| 11 | Cylinder adapter 15mm | DV9 | 1 |  |  | 11 | Cylinder adapter 15mm | DV9 | 1 |  |
| 12 | Bolt M8x25 | 815 | 2 |  |  | 12 | Bolt M8x25 | 815 | 2 |  |
| 13 | Washer M8 | 800 | 2 |  |  | 13 | Washer M8 | 800 | 2 |  |

LEFT SIDE INSTRUCTIONS

- Dismantle side fairing and bolt M12 from the frame.

- Fit poly nut ID7 using bolt ID5 on the adapter ID8.

- Stick tape ID9 on the adapter to prevent damage.

- Fit adapter using bolt ID2 and washer ID10. Place cylinder ID3 under adapter.

- Mount fairing back on the bike.

- Insert bolt ID4 through SLD adapter ID1, dural adapter ID8 and cylinder adapter ID11.

- Fit whole kit using poly nut ID6.

- Fit slider using bolts ID12 and washers ID13

We recommend to use safety glue on all threads (medium strong).

RIGHT SIDE INSTRUCTIONS

- Dismantle side fairing and bolt M12 from the frame.

- Fit poly nut ID7 using bolt ID5 on the adapter ID8.

- Stick tape ID9 on the adapter to prevent damage.

- Fit adapter using bolt ID2 and washer ID10. Place cylinder ID3 under adapter.

- Mount fairing back on the bike.

- Insert bolt ID4 through SLD adapter ID1, dural adapter ID8 and cylinder adapter ID11.

- Fit whole kit using poly nut ID6.

- Fit slider using bolts ID12 and washers ID13

We recommend to use safety glue on all threads (medium strong).