H14S

Honda CBR1000RR 2004-2005

| Left side: Right side: ID. | Description | Cat.No | Qty |
| --- | --- | --- | --- |
| 1 | Slider | SL01 | 1 |
| 2 | Bolt M12x90x1,25mm | 919 | 1 |
| 3 | Dural. cylinder d=18mm | 81 | 1 |
| 4 | Bolt M10x40 | 854 | 1 |
| 5 | Bolt M6x16mm | 962 | 1 |
| 6 | Poly nut M10 d=11mm | X7 | 1 |
| 7 | Poly nut M6 d=11mm | X8 | 1 |
| 8 | Duralumin adapter | 519 | 1 |
| 9 | Tape 3M |  | 1 |
| 10 | Pad M12 | 803 | 1 |
| 11 | Dural.adapter for SL | 313 | 1 |
| 12 | Bolt M4X16 | 876 | 2 |
| 13 | Bolt M6x25 | 877 | 3 |
| 14 | Cover | SLcap | 1 |

| ID. | Description | Cat.No | Qty |
| --- | --- | --- | --- |
| 1 | Slider | SL01 | 1 |
| 2 | Bolt M12x90x1,25mm | 919 | 1 |
| 3 | Dural. cylinder d=30mm | 82 | 1 |
| 4 | Bolt M10x40 | 854 | 1 |
| 5 | Bolt M6x16mm | 962 | 1 |
| 6 | Poly nut M10 d=11mm | X7 | 1 |
| 7 | Poly nut M6 d=11mm | X8 | 1 |
| 8 | Duralumin adapter | 519 | 1 |
| 9 | Tape 3M |  | 1 |
| 10 | Pad M12 | 803 | 1 |
| 11 | Dural.adapter for SL | 313 | 1 |
| 12 | Bolt M4X16 | 876 | 2 |
| 13 | Bolt M6x25 | 877 | 3 |
| 14 | Cover | SLcap | 1 |

Installation – the left side:

- remove the original side plastic and bolt M12 in frame of the motorcycle

- assembly polyamid cylinder ID.7 on the dural. Adapter, use bolt ID.5

- stick on back side of adapter tape 3M ID.9

- put duralumin cylinder ID.3 under adapter, use the bolt ID.2 with pad ID.10 and screw up adapter to frame of the motorcycle

- check all and then you can mount the original plastic side back

- gripp a dural.adapter for SL ID.11 on the adapter, use bolt ID.4 and poly. Nut ID.6

- make a assemlby slider ID.1, use bolts ID.13 and mount cover, use bolts ID.12

Installation – the right side:

- remove the original side plastic and bolt M12 in frame of the motorcycle

- assembly polyamid cylinder ID.7 on the dural. Adapter, use bolt ID.5

- stick on back side of adapter tape 3M ID.9

- put duralumin cylinder ID.3 under adapter, use the bolt ID.2 with pad ID.10 and screw up adapter to frame of the motorcycle

- check all and then you can mount the original plastic side back

- gripp a dural.adapter for SL ID.11 on the adapter, use bolt ID.4 and poly. Nut ID.6

- make a assemlby slider ID.1, use bolts ID.13 and mount cover, use bolts ID.12

Warning – we recommend to use safety glue on threads of all bolts (moderate strong – blue colour).