H15

Honda VTR1000 Firestorm 1997-

Fitting kit left side:		     		Fitting kit right side:

| ID | Description | Cat.N | Qty |
| --- | --- | --- | --- |
| 1 | Dural.adapter | 636 | 1 |
| 2 | Steel box | X1 | 1 |
| 3 | Dural.cylinder d=40 | 4 | 1 |
| 4 | Bolt M10x90 | 880 | 1 |
| 5 | Nut M12 | 811 | 1 |
| 6 | Washer M12 | 803 | 1 |
| 7 | 3M Tape |  | 1 |
| 8 | Steel bar 12x410x30x30 |  | 1 |

| ID | Description | Cat.N | Qty |
| --- | --- | --- | --- |
| 1 | Dural.adapter | 637 | 1 |
| 2 | Steel box | X1 | 1 |
| 3 | Dural.cylinder d=50 | 5 | 1 |
| 4 | Bolt M10x90 | 880 | 1 |
| 5 | Nut M12 | 811 | 1 |
| 6 | Washer M12 | 803 | 1 |
| 7 | 3M Tape |  | 1 |

Instruction to assembly:

- Dismantle original bolt from frame- M12.

- Through the frame insert stainless steel bar ID.8.

- Put on it adapter ID.1. Wher the adapter is touching with frame, stick 3M tape.

- Tighten adapter. Use bolt and washer ID.6, ID.5.

- Insert steel box ID.2 inside a protector, through the protector insert bolt ID.4 and put on it dural.cylinder ID.3.

- This set mount on the adapter.

Instruction to assembly-right side:

- Put on steel bar adapter ID.1. Wher the adapter is touching with frame, stick 3M tape.

- Tighten adapter. Use bolt and washer ID.6, ID.5.

- Insert steel box ID.2 inside a protector, through the protector insert bolt ID.4 and put on it dural.cylinder ID.3.

- This set mount on the adapter.

Warning: We recommend use the glue of all threads. Medium strong.