H18SD

Honda VFR 800 V-Tec

2004-2009

LEFT SIDE KIT				    	RIGHT SIDE KIT

| ID | Description | Cat.No | QT |
| --- | --- | --- | --- |
| 1 | Steel adapter | 515 | 1 |
| 2 | Steel adpater (short) | X48 | 1 |
| 3 | Alu. cylinder 10mm | DV124 | 1 |
| 4 | Alu. cylinder 25mm | DV88 | 1 |
| 5 | Alu. cylinder 27mm | DV123 | 1 |
| 6 | Alu. cylinder | DV125 | 2 |
| 7 | Bolt M10X50 | 849 | 1 |
| 8 | Bolt M8x50 | 837 | 1 |
| 9 | Bolt M6x60 | 872 | 1 |
| 10 | Bolt M6x30 | 863 | 2 |
| 11 | Washer M8 | 802 | 2 |
| 12 | Washer M10 | 804 | 2 |
| 13 | Washer M6 | 947 | 3 |
| 14 | Washer M6 (spring) | 985 | 2 |
| 15 | Nut M10 | 814 | 1 |
| 16 | Nut M8 | 813 | 1 |
| 17 | Washer M12 | 803 | 2 |
| 18 | Bolt 12(1,25)x395x30x30 |  | 1 |
| 19 | Steel nut | X12 | 1 |

| ID | Description | Cat.No | QT |
| --- | --- | --- | --- |
| 1 | Steel adapter | 516 | 1 |
| 2 | Steel adpater (short) | X76 | 1 |
| 3 | Alu. cylinder 25mm | DV122 | 1 |
| 4 | Alu. cylinder 15mm | DV121 | 1 |
| 5 | Alu. cylinder | DV125 | 2 |
| 6 | Bolt M10x30 | 882 | 1 |
| 7 | Bolt M8x40 | 828 | 1 |
| 8 | Bolt M6x60 | 872 | 1 |
| 9 | Bolt M6x30 | 863 | 2 |
| 10 | Washer M8 | 802 | 2 |
| 11 | Washer M10 | 804 | 2 |
| 12 | Washer M6 | 947 | 3 |
| 13 | Washer M6 (spring) | 985 | 2 |
| 14 | Nut M10 | 814 | 1 |
| 15 | Nut M8 | 813 | 1 |

PROTECTORS  FITTING KIT

| ID | Description | Cat.No | QT |
| --- | --- | --- | --- |
| 1 | SLD adapter | 537 | 2 |
| 2 | Dural. cylinder | DV9 | 2 |
| 3 | Šroub M8x25 | 815 | 4 |
| 4 | Washer M8 | 800 | 4 |
| 5 | Washer M10 (spring) | 801 | 2 |
| 6 | Bolt M10x40 | 854 | 2 |

We recommend to use medium strong blue to all threads.

MOUNTING KIT INSTRUCTIONS

- Dismantle side fairing of the motorcycle.

- Unbolt bolt M12 from the frame of motorcycle.

- Start on the right side first.

- Mount fitting kit on the bike according to the pictures below.

- Repeat this proces on the left side

- Stick 3M tape on the adapters to prevent damage of fairing.

- Mount fairing back on the motorcycle.

FITTING PROTECTORS

- Put washer ID5,  SLD adapter ID1 and dural. cylinder ID2 on the bolt ID6.

- Mount this kit into the steel adapter on the motorcycle.

- Fit protector using two bolts ID3.

3

5

12,13

13,14

8

5

9

6

1

18

4

17

1

9

15,10

2

6,11

12

10

7,12

7,10

11

14

13

15,12

4

2

3

8,11

11,16