H19

Honda VFR 800

1998-2002

| Kit for the left side |  |  |  |  | Kit for the right side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Description | Cat.no. | Pcs |  |  | No. | Description | Cat.no. | Pcs |  |
| 1 | Steel box M12 | X2 | 1 |  |  | 1 | Steel box M12 | X2 | 1 |  |
| 2 | Bolt M12x50mm | 869 | 1 |  |  | 2 | Bolt M12x50mm | 869 | 1 |  |
| 3 | Dural. cylinder d=70mm | 420 | 1 |  |  | 3 | Dural. cylinder d=60mm | 448 | 1 |  |
| 4 | Dural. cylinder d=11 | 17 | 1 |  |  | 4 | Dural. cylinder d=11 | 17 | 1 |  |
| 5 | Bolt M12x30mm | 855 | 1 |  |  | 5 | Bolt M12x30mm | 855 | 1 |  |
| 6 | Steel adapter with bolt | 621 | 1 |  |  | 6 | Steel adapter | 622 | 1 |  |
| 7 | 3M tape |  | 1 |  |  | 7 | Nut M12-safety | 811 | 1 |  |
|  |  |  |  |  |  | 8 | Pad M12-plane | 803 | 1 |  |
|  |  |  |  |  |  | 9 | 3M tape |  | 1 |  |

Installation:

- remove the complete side cowl and the bottle of cooling water on the left side

- remove the original bolt M12

- be careful, there is a duralumin cylinder between the frame and the engine on the right side, the cylinder falls probably after removing of the bolt, you have to give a cylinder on the right place at tightening

- use the bolt (No. 5)  for mounting of duralumin cylinder (No. 3) on the left steel adapter (No. 6) with bolt

- give on bolt, which is welded with steel adapter, duralumin cylinder (No. 4)

- all this set give into the frame of motorcycle on the left side, use the tape 3M (No. 7) at the place of contact of the steel adapters and frame of the motorcycle

- use the bolt (No. 5) for mounting of duralumin cylinder (No. 3) on right steel adapter (No. 6)

- on the right side give duralumin cylinder (No. 4) and steel adapter (No. 6) on the bolt, use the tape 3M (No. 7) at the place of contact of the steel adapters and frame of the motorcycle

- use the nut (No. 7) with the pad (No. 8) for tightening the complete set

- during installation you will need to drill a hole with a diameter 31mm in the original cowl

- finish the installation of complete fitting kit and mount the side cowl back, take the crash protectors and give the bolts (No. 2) with steel boxes (No. 1) into the protector, all this set mount into the duralumin cylinders (No. 3)

Warning – we recommend to use safety glue on threads of all bolts (moderate strong – blue colour).