H1

Honda CBF500 2004-

Left side fitting kit:                                          Right side fitting kit:

| ID. | Description | Cat.no. | Qt |
| --- | --- | --- | --- |
| 1 | Steel box M10 | X1 | 1 |
| 2 | Steel nut M10 | X11 | 1 |
| 3 | Duralumin cylinder d=40mm | 4 | 1 |

| ID. | Description | Cat.no. | Qt |
| --- | --- | --- | --- |
| 1 | Steel box M10 | X1 | 1 |
| 2 | Steel nut M10 | X11 | 1 |
| 3 | Duralumin cylinder d=40mm | 4 | 1 |
|  | Stainless steel bar 10(1,25)x445x30x30 |  | 1 |

Installation:

- Dismantle the long original bolt M10, as you can see on the picture.

- Inser the stainless steel bar M10x450x1,25mm through the engine and frame.

- Place duralumin cylinder (ID. 3) on this stainless steel bar both sides.

- Insert steel box (ID. 1) into the protector and now place protectors on stainless steel bar too

- Finally screw up all with nuts M12 (ID. 2)

Warning – we recommend to use safety glue on threads of all bolts (moderate strong – blue colour).