H22SD

Honda CB1000R Hornet 2008-2011

Fitting kit:

| ID. | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Slider holder | 537 | 2 |
| 2 | Spacer d=15mm | DV-149 | 2 |
| 3 | Bolt M12x95x1,25 | 900 | 2 |
| 4 | Bolt M8x25 | 815 | 4 |
| 5 | Washer | 802 | 4 |

Assembly instruction are same for both sides.

- Dismantle original bolts M12 from chassi.

- Insert bolt ID.3 into the slider holder and spacer. Fix it into the engine thread.

- Place slide rand fix it by bolts + washers M8.

Note: We recommend use medium strong glue to all threads.