H26

Honda CBR600RR 2009-

Kit for left side:		     		                   Kit for right side:

| ID | Description | Cat.N | QTY |
| --- | --- | --- | --- |
| 1 | Steel box M12 | X2 | 1 |
| 2 | Bolt M12x110x1,25 | 910 | 1 |
| 3 | Dural.cylinder d=31,5 | 288 | 1 |

2

1

2

3

1

3

| ID | Description | Cat.N | QTY |
| --- | --- | --- | --- |
| 1 | Steel box M12 | X2 | 1 |
| 2 | Bolt M12x90x1,25 | 931 | 1 |
| 3 | Dural.cylinder d=16,5 | 289 | 1 |

You will have to drill the hole in the fairing during installation. It is therefore necessary intervention in the original cowl!

Take out fairing from bike and make a hole.

Instruction to assembly left side:

- Take out original bolt from frame.

- Insert steel box ID.1 into the protector. Under the protector insert dural.cylinder ID.3. This set gripping on the frame.

Instruction to assembly left side:

- Take out original bolt from frame.

- Insert steel box ID.1 into the protector. Under the protector insert dural.cylinder ID.3. This set gripping on the frame.

Warning: We recommend use the glue of all threads. Medium strong.