H27

Honda CBF1000 2010-

| Kit for the left side |  |  |  |  | Kit for the right side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID. | Description | Cat.no. | Pcs |  |  | ID. | Description | Cat.no. | Pcs |  |
| 1 | Steel box M10 | X1 | 1 |  |  | 1 | Steel box M10 | X1 | 1 |  |
| 2 | Bolt M10x80mm | 852 | 1 |  |  | 2 | Bolt M10x80mm | 852 | 1 |  |
| 3 | Dural. cylinder d=21mm | 256 | 1 |  |  | 3 | Dural. cylinder d=21mm | 256 | 1 |  |
| 4 | Nut M10 (safety) | 810 | 1 |  |  | 4 | Nut M10 (safety) | 810 | 1 |  |

Installation:

- Installation of the left and right protector make in gripping about the attached photo above. Give the steel box M10 (ID. 1) on both bolt.

- Put the duralumin cylinder (ID. 3) under the protector on the left side and use the bolt (ID. 2) for tightening.

- Put the duralumin cylinder (ID. 3) under the protector on the right side and use the bolt (ID. 2) for tightening.

- At first finish the one side and then mount the second side.

Warning – we recommend to use safety glue on threads of all bolts (moderate strong – blue colour).