H28

Honda VTR250 2010-

| Kit for the left side |  |  |  |  | Kit for the right side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Description | Cat.no. | Pcs |  |  | No. | Description | Cat.no. | Pcs |  |
| 1 | Steel box M12 | X2 | 1 |  |  | 1 | Steel box M12 | X2 | 1 |  |
| 2 | Bolt M12x160x1,25mm | 949 | 1 |  |  | 2 | Bolt M12x120x1,25mm | 913 | 1 |  |
| 3 | Dural. cylinder d=23,5mm | 258 | 1 |  |  | 3 | Dural. cylinder d=23,5mm | 258 | 1 |  |

Installation:

- Installation of the left and right protector make in gripping about the attached photo above.

- Give the steel box M12 (No. 1) on both bolt. Put the duralumin cylinder (No. 3) under the protector on the left side and use the bolt (No. 2) for tightening. Put the duralumin cylinder (No. 3) under the protector on the right side and use the bolt (No. 2) for tightening.

- At first finish the one side and then mount the second side.

Warning – we recommend to use safety glue on threads of all bolts (moderate strong – blue colour).