H2SD

Honda CB600F Hornet 1998-2006

Honda CBF600(S) 2004-2007

Kit for left side:		     		        Kit for right side:

| ID | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Dural.bracket for SLD | 537 | 1 |
| 2 | Bolt M8x25 | 815 | 2 |
| 3 | Washer M8 | 800 | 2 |
| 4 | Dural. spacer d=40mm | 12 | 1 |
| 5 | Bolt M10x120 | 881 | 1 |
| 6 | Washer M10 | 804 | 1 |
| 7 | Nut M10 | 810 | 1 |

| ID | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Dural.bracket for SLD | 537 | 1 |
| 2 | Bolt M8x25 | 815 | 2 |
| 3 | Washer M8 | 800 | 2 |
| 4 | Dural. spacer d=15mm | 9 | 1 |
| 5 | Bolt M10x90 | 880 | 1 |
| 6 | Washer M10 | 804 | 1 |
| 7 | Nut M10 | 810 | 1 |

1

1

4

6

6

7

7

4

5

5

2

2

3

3

Instruction to assembly left side:

- Take out original bolt M10 from frame.

- On the bolt ID.5 insert dural. bracket for sld ID.1. Between the dural bracket and frame put dural. spacer ID.4 This kit mounting on the bike, from back side tighten bolt by washer ID.6 and nut ID.7.

- On the bolts ID.2 put washers ID.3 and mounting slider.

Instruction to assembly right side:

- Take out original bolt M10 from frame.

- On the bolt ID.5 insert dural. bracket for sld ID.1. Between the dural bracket and frame put dural. spacer ID.4 This kit mounting on the bike, from back side tighten bolt by washer ID.6 and nut ID.7.

- On the bolts ID.2 put washers ID.3 and mounting slider.

Use glue of all threads. Medimum strong. (blue colour).