H2

Honda CBF600 (S) 2004-2007

Honda CB600F Hornet 1998-2006

| Kit for the left side |  |  |  |  | Kit for the right side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Description | Cat.N | Pcs |  |  | No. | Description | Cat.N | Qty |  |
| 1 | Steel box M10 | X1 | 1 |  |  | 1 | Steel box M10 | X1 | 1 |  |
| 2 | Bolt M10x110mm | 879 | 1 |  |  | 2 | Bolt M10x95mm | 822 | 1 |  |
| 3 | Dural. cylinder d=15mm | 8 | 1 |  |  | - | -------------- | -- | - |  |
| 4 | Nut M10 (safety) | 810 | 1 |  |  | 4 | Nut M10 (safety) | 810 | 1 |  |
| 5 | Washer M10 | 804 | 1 |  |  | 5 | Washer M10 | 804 | 1 |  |

Installation:

- Installation of the left and right protector make in gripping about the attached photo above.

- Left side: Give the steel box M10 (No. 1) on the bolt. Put the duralumin cylinder (No. 3) under the protector on the left side and use the bolt (No. 2) for tightening use washer No.5 and nut No.4 from back side.

- Right side: Bolt No.2 put through steel box No.1 and protector. Insert this bolt into the frame and engine and mounting it from back side by washer No.5 and nut No.4

- At first finish the one side and then mount the second side.

Warning – we recommend to use safety glue on threads of all bolts (moderate strong – blue colour).