- H33S

- Honda CBR600F 2011-

| Kit for the left side: |  |  |  |  | Kit for the right side: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.No | QTY |  |  | ID | Description | Cat.No | QTY |  |
| 1 | Bolt M6x25 | 877 | 3 |  |  | 1 | Bolt M6x25 | 877 | 3 |  |
| 2 | Bolt M12x80x1,25mm | 908 | 1 |  |  | 2 | Bolt M12x70x1,25mm | 937 | 1 |  |
| 3 | Bolt M10x60x1,25mm | 896 | 1 |  |  | 3 | Bolt M10x60x1,25mm | 896 | 1 |  |
| 4 | Bolt M10x60mm | 850 | 1 |  |  | 4 | Bolt M10x60mm | 850 | 1 |  |
| 5 | Dural. cylinder d=15mm | 455 | 1 |  |  | 5 | Dural. cylinder d=16,5mm | 454 | 1 |  |
| 6 | Dural. cylinder d=5mm | 456 | 1 |  |  | 6 | Dural. cylinder d=4mm | 79 | 1 |  |
| 7 | Steel adapter | 624 | 1 |  |  | 7 | Steel adapter | 625 | 1 |  |
| 8 | Washer spring M10 | 801 | 1 |  |  | 8 | Washer spring M10 | 801 | 1 |  |
| 9 | Tape 3M |  | 1 |  |  | 9 | Tape 3M |  | 1 |  |
| 10 | Nut M10 | 810 | 1 |  |  | 10 | Nut M10 | 810 | 1 |  |
| 11 | Bolt M4x16 | 876 | 2 |  |  | 11 | Bolt M4x16 | 876 | 2 |  |
| 12 | Dural. adapter for SL01 | 339 | 1 |  |  | 12 | Dural. adapter for SL01 | 339 | 1 |  |

6

3

2

5

6

7

10

9

4

1

8

5

5

3

3

2

6

6

2

7

1

11

7

4,8

12

11

10

9

1

10

12

4

8

- Instruction to assembly-left side:

- Take off side fairing, bolt M10 and M12 from frame.

- Through adapter ID.7 insert bolts ID.2 and ID.3.

- Under the bolts put dural. cylinders ID.5 a ID.6. from back side the adapter.

- This complete set mounted on the bike.

- During the assembly you can stick 3M tape under the adapter.

- Bolt ID.4 with washer ID.8 insert into the dural adapter ID.12

- This set mounted on the adapter, for fixed use nut ID.10.

- Slider gripping on the dural adapter by bolts ID.1, fit cover inot slider by bolts ID.11

- Put fairing back on bike.

- Instruction to assembly-right side:

- Take off side fairing, bolt M10 and M12 from frame.

- Through adapter ID.7 insert bolts ID.2 and ID.3.

- Under the bolts put dural. cylinders ID.5 a ID.6. from back side the adapter.

- This complete set mounted on the bike.

- During the assembly you can stick 3M tape under the adapter.

- Bolt ID.4 with washer ID.8 insert into the dural adapter ID.12

- This set mounted on the adapter, for fixed use nut ID.10.

- Slider gripping on the dural adapter by bolts ID.1, fit cover inot slider by bolts ID.11

- Put fairing back on bike.

Warning – on the threads of the screws is recommended to use premium adhesive moderate (blue color).