H34SDM

Honda CBR125R 2011-

| Kit for the left side |  |  |  |  | Kit for the right side |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.no. | Pcs |  | ID | Description | Cat.no. | pcs |
| 1.1 | Steel adapter - black | 507 | 1 |  | 1.2 | Steel adapter - black | 506 | 1 |
| 2.1 | Dural cylinder – 80mm | 1195 | 1 |  | 2.2 | Dural cylinder - 95mm | 1196 | 1 |
| 3.1 | Screw M8x45mm | 820 | 2 |  | 3.2 | Screw M8x25mm | 833 | 2 |
| 4 | Steel connection rod | 8x300x15x15 | 1 |  | 4 | - | - | - |
| 5 | Nut M8 | 809 | 1 |  | 5 | Nut M8 | 809 | 1 |
| 6 | Washer M8 | 802 | 1 |  | 6 | Washer M8 | 802 | 1 |
| 7 | Nut cover M8 | X46 | 1 |  | 7 | Nut cover M8 | X46 | 1 |
| 8 | Bolt M10x95 | 822 | 1 |  | 8 | Bolt M10x95 | 822 | 1 |
| 10 | Dural cylinder - 70mm | 15 | 1 |  | 10 | Dural cylinder - 70mm | 15 | 1 |
| 11 | Nut M10 | 810 | 1 |  | 11 | Nut M10 | 810 | 1 |
| 12 | Washer M10 | 804 | 1 |  | 12 | Washer M10 | 804 | 1 |
| 13 | Bolt M6x20 | 1024 | 2 |  | 13 | Bolt M6x20 | 1024 | 2 |
| 14 | Washer M6 | 1023 | 2 |  | 14 | Washer M6 | 1023 | 2 |
| 15 | SLDM adapter | 537/B | 1 |  | 15 | SLDM adapter | 537/B | 1 |

ID3.2

ID3.1

ID13+14

ID1.2

ID1.1

ID10

ID15

ID11+12

ID8+12

ID4

ID5

ID6

ID2.1

ID2.2

ID7

- Remove fairing from both sides.

Instructions for the left side:

- Remove original bolts.

- Fit steel adapter (ID1.1), using bolts (ID3.1) on the frame.

- Insert dural cylinder (ID2.1).

- Insert steel rod M8 (ID4) through adapter, dural cylinder and engine.

- Tighten steel rod M8 using washer (ID6) and nut (ID5).

- Place plastic cover (ID7).

- Fit SLD adapter (ID15) with dural cylinder (ID10) using bolt (ID8+ID12) and nut (ID11+ID12)

Instructions for the protector:

- Fit protector on the SLD adapter using two bolts ID34 and washers ID14.

(illustrative photo of different protector)

Montáž pravá strana:

Right side:

(Almoast the same as left.)

Differences:

- Insted of adapter ID1.1 use adapter ID1.2

- Insted of dural cylinder ID2.1 use dural cylinder ID2.2

- Insted of bolts ID3.1 use bolts ID3.2

In the end put fairing back.

We recommend to use the glue on all threads (medium strong).