H34S

Honda CBR125R 2011-

| Kit for left side: |  |  |  |  | Kit for right side: |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.no. | pcs |  | ID | Description | Cat.no. | pcs |  |
| 1.1 | Steel protector holder - black | 507 | 1 |  | 1.2 | Steel protector holder - black | 506 | 1 |  |
| 2.1 | Dural roller - 80mm | 1195 | 1 |  | 2.2 | Dural roller - 95mm | 1196 | 1 |  |
| 3.1 | screw M8x45mm | 820 | 2 |  | 3.2 | screw M8x25mm | 833 | 2 |  |
| 4 | Steel connection rod | 8x300x15x15 | 1 |  | - | - | - | - |  |
| 5 | nut M8 | 809 | 1 |  | 5 | Nut M8 | 809 | 1 |  |
| 6 | Washer M8 | 802 | 1 |  | 6 | washer M8 | 802 | 1 |  |
| 7 | Nut cover M8 | X46 | 1 |  | 7 | Nut cover M8 | X46 | 1 |  |
| 8 | Slider RDmoto | SL | 1 |  | 8 | Slider RDmoto | SL | 1 |  |
| 9 | cover | SLcap. | 1 |  | 9 | cover | SLcap. | 1 |  |
| 10 | Slider adapter - 40mm | 317 | 1 |  | 10 | Slider adapter - 40mm | 317 | 1 |  |
| 11 | screw M10x80 | 852 | 1 |  | 11 | screw M10x80 | 852 | 1 |  |
| 14 | screw M6x25 | 877 | 3 |  | 14 | screw M6x25 | 877 | 3 |  |
| 15 | screw M4x16 | 876 | 2 |  | 15 | screw M4x16 | 876 | 2 |  |
| 16 | Dural roller d=50mm | 5 | 1 |  | 16 | Dural roller d=50mm | 5 | 1 |  |
| 17 | Nut M10 | 810 | 1 |  | 17 | Nut M10 | 810 | 1 |  |
| 18 | Washer M10 | 804 | 1 |  | 18 | Washer M10 | 804 | 1 |  |

ID1.2

ID4

ID14

ID3.2

ID1.1

ID1

ID7

ID2.1

ID2.2

ID3.1

ID5

ID6

ID10

ID11+17+18

ID15

Assembly left side:

- Remove fairing from both sides.

- Dismantle original bolts from chassi.

- Place steel adapter (ID1.1) and tighten it by bolts (ID3.1).

- Place dural roller (ID2.1).

- Insert connection bar M8 (ID4). through the steel bracket, dural roller and engine.

- Use nut (ID5) and washer (ID6) for tightening.

- Place plastic cover (ID7).

(right hand side)

Instructions for slider:

- Place slider adapter (ID10) on steel adapter with screw M10 (ID11) washer (ID17) and nut (ID18).

- Adjut axis of slider before instalation.

- Tighten screw M10 (ID11).

- Tighten slider (ID8) with 3 screws M6 (ID14).

- Insert alu cover (ID9) with 2 screws (ID15).

Right side assembly:

(almoast the same as on the left side)

There are just few diferences:

- Instead of adapter ID1.1 use ID1.2

- Instead of dural roller ID2.1 use ID2.2

- Instead of screws ID3.1 use screws ID3.2

Place fairing back the bike.

We recommande use medium strong glue for all threads during assembly.