H36S

Honda CBR1000RR 2012-

Without ABS

| Kit for the left side: |  |  |  |  | Kit for the right side: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.no. | pcs |  |  | ID | Description | Cat.no. | pcs |  |
| 1 | Slider RDmoto | SL | 1 |  |  | 1 | Slider RDmoto | SL | 1 |  |
| 2 | Coloured cover | SLcap.. | 1 |  |  | 2 | Coloured cover | SLcap.. | 1 |  |
| 3 | Dural adapter | DV-339 | 1 |  |  | 3 | Dural adpater | DV-339 | 1 |  |
| 4 | Bolt M6x25 | 877 | 3 |  |  | 4 | Bolt M6x25 | 877 | 3 |  |
| 5 | Bolt M4x16 | 876 | 2 |  |  | 5 | Bolt M4x16 | 876 | 2 |  |
| 6 | Bolt M10x50mm | 849 | 1 |  |  | 6 | Bolt M10x50mm | 849 | 1 |  |
| 7 | Bolt M8x25mm | 832 | 1 |  |  | 7 | Bolt M8x25mm | 832 | 1 |  |
| 8 | Steel adapter | 644 | 1 |  |  | 8 | Steel adapter | 645 | 1 |  |
| 9 | Dural spacer d=11mm | 17 | 1 |  |  | 9 | Dural spacer d=4mm | 119 | 1 |  |
| 10 | Tape 3M |  | 1 |  |  | 10 | tape 3M |  | 1 |  |
| 11 | Washer M12 | 803 | 1 |  |  |  |  |  |  |  |
| 12 | Bolt M12x70x1,25mm | 917 | 1 |  |  |  |  |  |  |  |
| 13 | Steel rod 8x463x30x30 |  | 1 |  |  |  |  |  |  |  |

Installation:

- Remove both side fairing of motorcycle.

- Remove original M12 bolts.

- Place protective tape between frame and adapter.

- Fit steel adpaters on both sides according to the picture below. Use original bolt for tightening. Don’t forget to insert dural spacer (ID9) between adapter and frame.

- Tighten left side adapter using bolt (ID12).

- Insert steel rod (ID13) between adapters and tighten it using bolts (ID7).

- Now mount fairing back on the motorcycle.

- Installation of sliders:

  - Fit dural adapter (ID3) on the steel adapter using bolt M10(ID6).

  - Set angle of this adapter.

  - Tighten bolt M10.

  - Fit slider on slider adapter and tighten it using three bolts M6 (ID5).

  - In the end fit coloured cover into slider using two bolts M4 (ID4).

- We recommend to use the glue on all threads

8

9

8

5

9

1

4

5

3

1

11

12

6

6

3

4

10

2

13

7

7