H37SDM

Honda Crosstourer

Mounting Kit Left Side:		     		Mounting Kit Right Side:

| ID | Description | Cat.No. | Pcs |
| --- | --- | --- | --- |
| 1 | Dural. roller | 149 | 1 |
| 2 | Dural. adapter | 1124 | 1 |
| 3 | Bolt M12x160x1,25 | 949 | 1 |
| 4 | Platform SLDM | 537/B | 1 |
| 5 | Bolt M6x20 | 1024 | 2 |
| 6 | Washer M6 | 1023 | 2 |

| ID | Description | Cat.No | Pcs |
| --- | --- | --- | --- |
| 1 | Dural. roller | 149 | 1 |
| 2 | Dural. adapter | 1124 | 1 |
| 3 | Bolt M12x150x1,25 | 984 | 1 |
| 4 | Platform SLDM | 537/B | 1 |
| 5 | Bolt M6x20 | 1024 | 2 |
| 6 | Washer M6 | 1023 | 2 |

1

6

2

4

5

3

1

3

Montáž levá strana:

- Unscrew the original M12 bolt from frame.

- Put the ID.4 platform onto ID.3 bolt with ID.1 dural. roller and ID.2 dural. adapter.

- Now place this assembly in place of the original bolt.

- Finally, use ID.5 bolts and ID.6 washers to fix the slider.

Montáž pravá strana:

- Unscrew the original M12 bolt from frame.

- Put the ID.4 platform onto ID.3 bolt with ID.1 dural. roller and ID.2 dural. adapter.

- Now place this assembly in place of the original bolt.

- Finally, use ID.5 bolts and ID.6 washers to fix the slider.

Note – We recommend using thread glue during assembly.