H38S

Honda CBR1000RR 2012-

WITH ABS

| Kit for the left side |  |  |  |  | Kit for the right side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Description | Cat.no. | Pcs |  |  | No. | Description | Cat.no. | Pcs |  |
| 1 | Slider RDmoto | SL | 1 |  |  | 1 | Slider RDmoto | SL | 1 |  |
| 2 | Cover | SLcap.. | 1 |  |  | 2 | Cover | SLcap.. | 1 |  |
| 3 | Adapter slider | DV-339 | 1 |  |  | 3 | Adapter slider | DV-339 | 1 |  |
| 4 | Bolt M6x25 | 877 | 3 |  |  | 4 | Bolt M6x25 | 877 | 3 |  |
| 5 | Bolt M4x16 | 876 | 2 |  |  | 5 | Bolt M4x16 | 876 | 2 |  |
| 6 | Bolt M8x25mm | 832 | 1 |  |  | 6 | Bolt M8x25mm | 832 | 1 |  |
| 7 | Bolt M10x60mm | 850 | 1 |  |  | 7 | Bolt M10x60mm | 850 | 1 |  |
| 8 | Steel adapter | 644 | 1 |  |  | 8 | Steel adapter | 645 | 1 |  |
| 9 | Dural. cylinder d=11mm | 17 | 1 |  |  | 9 | Dural. cylinder d=1mm | 472 | 1 |  |
| 10 | Tape 3M |  | 1 |  |  | 10 | Tape 3M |  | 1 |  |
| 11 | Pad M12 | 803 | 1 |  |  |  |  |  |  |  |
| 12 | Bolt M12x70x1,25mm | 917 | 1 |  |  |  |  |  |  |  |
| 13 | Steel bar 8x463x30x30 |  | 1 |  |  |  |  |  |  |  |
| 14 | Steel holder to transfer of ABS | X16 | 1 |  |  |  |  |  |  |  |

Installation:

1. Remove the complete side cover

2. Remove the original screw M12
3. For mounting of steel adapters use to define the space between the adapter and the motorcycle frame 3M tape
4. On the left and right side of the motorcycle frame attach steel adapters. Right adapter insert into the motorcycle frame and by the original screw tighten. Do not forget to insert duralumin cylinder spacer between the adapter and the motorcycle frame (item 9) and new adapter for ABS (item 14)
5. Left adapter tighten by the screw (item 12), do not forget to insert duralumin cylinder between the adapter and the motorcycle frame  (item 9)
6. Between the left and right adapter insert a stainless steel rod (item 13), tighten by the screws (item 6)
7. Tighten everything thoroughly and can be mounted side plastics
8. Now start to mount attachment for left slider. On the steel adapter mount by screw (item 7) duralumin adapter (item 3)
9. Insert the slider (item 1) and fix by the screws (item 4)
10. Insert the cover to the slider (item 2) and fix by the screws (item 5)
11. Mounting of the slider on the right side is the same

Use a steel holder to transfer of the ABS during installation.

Warning – we recommend to use safety glue on threads of all bolts (moderate strong – blue colour).