H3SD

Honda CB900F Hornet 2002-

| Kit for the left side |  |  |  |  | Kit for the right side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Description | Cat.No. | Pcs |  |  | No. | Description | Cat.No. | Pcs |  |
| 1 | SLD adapter | 537 | 1 |  |  | 1 | SLD adapter | 537 | 1 |  |
| 2 | Bolt M10x85mm | 981 | 1 |  |  | 2 | Bolt M10x85mm | 981 | 1 |  |
| 3 | Dural. cylinder 30mm | 3 | 1 |  |  | 3 | Dural. cylinder 30mm | 3 | 1 |  |
| 4 | Spring Washer M10 | 801 | 1 |  |  | 4 | Spring Washer M10 | 801 | 1 |  |
| 5 | Bolt M8x25 | 815 | 2 |  |  | 5 | Bolt M8x25 | 815 | 2 |  |
| 6 | Washer M8 | 800 | 2 |  |  | 6 | Washer M8 | 800 | 2 |  |
| 7 | Dural. adapter (cylinder) | 10 | 1 |  |  | 7 | Dural. adapter (cylinder) | 10 | 1 |  |

Instructions

- Unbolt original bolt from the frame of motorcycle.

- Put washer ID4, SLD adapter ID1, dural cylinder ID3 and cylinder adapter ID7 on bolt ID2.

- Mount this kit into the frame of motorcycle.

- Fit slider on the SLD adapter using two bolts ID5 and washers ID6.

We recommend to use medium strong glue on all threads.