H3S

Honda CB900 F Hornet 2002-

Fitting kit left side:		     		       Fitting kit right side:

| ID | Description | Cat.no | QTY |
| --- | --- | --- | --- |
| 1 | Dural.adapter | 318 | 1 |
| 2 | screw M10x35 | 847 | 1 |
| 3 | screw M6x25 | 877 | 3 |
| 4 | Screw M4x16 | 876 | 2 |

| ID | Description | Cat.no | QTY |
| --- | --- | --- | --- |
| 1 | Dural.adapter | 318 | 1 |
| 2 | screw M10x35 | 847 | 1 |
| 3 | screw M6x25 | 877 | 3 |
| 4 | Screw M4x16 | 876 | 2 |

Instruction to assembly:

- Installation of the protector on the right and left side make to the mounting, where joins the motorcycle frame with engine.

- On the motorcycle frame is prepared hole with a diameter of 8mm, this hole resize drill 8.5mm and make thread M10.

- Under the protector on the right and left side insert distance duralumin adapter ID.1 and by screw ID.2 tighten it.

- Make assembly the sliders on this adapters. Use bolts ID.3.

- Take aluminum cover, insert are into slide rand tighten are by bolts ID.4.

Warning: We recommend use the glue of all threads. Medium strong.