H3

Honda CB900F Hornet 2002-

| Kit for Left Side |  |  |  |  | Kit for Right Side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.No. | Pcs |  |  | ID | Description | Cat.No. | Pcs |  |
| 1 | Steel Insert M10 | X1 | 1 |  |  | 1 | Steel Insert M10 | X1 | 1 |  |
| 2 | Bolt M10x80mm | 852 | 1 |  |  | 2 | Bolt M10x80mm | 852 | 1 |  |
| 3 | Dural. roller d=30mm | 3 | 1 |  |  | 3 | Dural. roller d=30mm | 3 | 1 |  |

Mounting Instructions:

- Mount both sides of the protector following instructions on the below attached photographs.

- Where there are plastic covers on the frame, take them off and drill this hole with drill of diameter 8,4mm.

- Drill an M10 thread into the created hole.

- Equip both bolts with steel inserts (ID.1) Now insert the dural. roller (ID.3) under the protector on left side and tighten with ID.2 bolt. Repeat the same on right side with ID.3 roller and ID.2 bolt.

- Finish one side completely before proceeding with the other one.

Note – We recommend using medium strength glue for threads of all bolts (blue color).