H4-2

Honda CBR1000RR 2006-2007

| Kit for the left side |  |  |  |  | Kit for the right side |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Description | Cat.no. | Pcs |  | No. | Description | Cat.no. | Pcs |  |
| 1 | Steel box M12 | X2 | 1 |  | 1 | Steel box M12 | X2 | 1 |  |
| 2 | Bolt M12x100x1,25mm | 911 | 1 |  | 2 | Bolt M12x100x1,25mm | 911 | 1 |  |
| 3 | Dural. cylinder d=26,5mm | 139 | 1 |  | 3 | Dural. cylinder d=22mm | 146 | 1 |  |

Installation:

You will have to drill the hole in the cowl during installation. It is therefore necessary intervention in the original cowl!

Installation of the left and right protector make in gripping about the attached photo above. Give the steel box M12 (No. 1) on both bolts. Put the duralumin cylinder (No. 3) under the protector on the left side and use the bolt (No. 2) for tightening. Put the duralumin cylinder (No. 3) under the protector on the right side and use the bolt (No. 2) for tightening. At first finish the one side and then mount the second side.

Warning – we recommend to use safety glue on threads of all bolts (moderate strong – blue colour).