H40S

Honda CBR500R, 2013-15

Kit for the left side:		     		       Kit for the right side:

| ID | Description | Cat.no | pcs |
| --- | --- | --- | --- |
| 1 | Dural roller - 90mm | 4 | 1 |
| 2 | Dural adapter | 317 | 1 |
| 3 | Screw M10x70 | 851 | 1 |
| 4 | Steel protector holder | 660 | 1 |
| 5 | Screws M6x35mm | 989 | 2 |
| 6 | Screw M10x65x1.25mm | 945 | 1 |
| 7 | Insert M10 | 804 | 2 |
| 8 | ALU roller d=19,5mm | 1121 | 1 |
| 9 | Nut M10 | 810 | 1 |
| 10 | Screw M6x25 | 877 | 3 |
| 11 | Screw M4x16 | 876 | 2 |

| ID | Description | Cat.no | pcs |
| --- | --- | --- | --- |
| 1 | Dural roller – 40mm | 4 | 1 |
| 2 | Dural adapter | 317 | 1 |
| 3 | Screw M10x70 | 851 | 1 |
| 4 | Steel protector holder | 661 | 1 |
| 5 | Screws M6x35mm | 989 | 2 |
| 6 | Screw M10x100x1.25mm | 892 | 1 |
| 7 | Insert M10 | 804 | 2 |
| 8 | ALU roller d=32mm | 1120 | 1 |
| 9 | Nut M10 | 810 | 1 |
| 10 | Screw M6x25 | 877 | 3 |
| 11 | Screw M4x16 | 876 | 2 |

Installation (left side):

- Remove fairing from motorcycle.

- Remove original screws according to the picture.

- Attach steel adapter ID.4 using screws ID.5.

- Between frame and steel adapter insert aluminium roller ID.8. Tighten adapter using screw M10x65 and insert (ID.7 and ID.6).

- Place adapter ID2 on screw ID3 and place also durall roller ID1 under adapter.

- This set attach on steel adapter on motorcycle.

- Fit nut and insert M10 from the backside.

- Fit slider with screws ID10 and isert cover.

- Tighten colored cover using screws ID11.

Installation (right side:

Almost the same as on the left except one diference:

- Dimensions of dural roller and screw M10x100x1.25.

We recommend: use the glue on all threads (medium strong).

3,9,7

2

3,9,7

1

5

11

10

6

7

1

5

4

8

6

2

5

5

8

10

11

2

4