H41S

Honda CBR 600RR-´14

Fitting kit left side:		     		Fitting kit right side:

| ID | Description | Cat.n | Qt |
| --- | --- | --- | --- |
| 2 | Dural.adapter | DV317 | 1 |
| 3 | Dural. spacer | 1122 | 1 |
| 4 | Bolt M10x50 | 849 | 1 |
| 5 | Steel adapt. | 668 | 1 |
| 6 | Washer M12 | 803 | 1 |
| 7 | Bolt M12x80x1,25 | 891 | 1 |
| 8 | Alu.spacer | 1123 | 1 |
| 9 | 3M Tape | 3M | 2 |
| 10 | Bolt M6x25 | 877 | 3 |
| 11 | Bolt M4x16 | 876 | 2 |

| ID | Popis | Kat.č. | Ks |
| --- | --- | --- | --- |
| 2 | Dural.adapter | DV317 | 1 |
| 3 | Dural. spacer | 1122 | 1 |
| 4 | Bolt M10x50 | 849 | 1 |
| 5 | Steel adapter | 669 | 1 |
| 6 | Washer M12 | 803 | 1 |
| 7 | Bolt M12x80x1,25 | 891 | 1 |
| 8 | Alu. spacer | 1123 | 1 |
| 9 | 3M Tape | 3M | 2 |
| 10 | Bolt M6x25 | 877 | 3 |
| 11 | Bolt M4x16 | 876 | 2 |

Fitting instruction:

- Dismantle lower fairing from your Honda.

- Start to assembly left side first and after this done right side. Process is same.

- Take off oringal bolt M12 from frame.

- Stick 3M tape on the frame ID.9, mounting steel adapter ID.5 from out of frame. Insert alu spacer ID.8 Tighten it by bolt ID.7 and washer ID.6.

- Make same process on the right side.

- Put fairing back the bike.

- Insert bolt ID.4 into the dural.adapter ID.2. Place spacer ID.3. This kit mount on the steel adapter.By bolts ID.10 mount slider,place cover ant use bolts ID.11.

Note: During installation, we recommend using glue on the threads.

13

12

11

10

9

8

7

6

5

4

3

2

2

10

11

3

4

5

6

7