H41

Honda CBR 600RR-´14

Fitting kit left side:		     		   Fitting kit right side:

| ID | Description | Cat.n | Qty |
| --- | --- | --- | --- |
| 2 | Steel box | X1 | 1 |
| 3 | Dural.reduction cyli. | 1122 | 1 |
| 4 | Bolt M10x60 | 850 | 1 |
| 5 | Steel adapter | 668 | 1 |
| 6 | Washer M12 | 803 | 1 |
| 7 | Bolt M12x80x1,25 | 891 | 1 |
| 8 | Aluminium cylinder | 1123 | 1 |
| 9 | 3M Tape | 3M | 2 |

| ID | Description | Cat.n | Qty |
| --- | --- | --- | --- |
| 2 | Steel box | X1 | 1 |
| 3 | Dural.reduction cyli. | 1122 | 1 |
| 4 | Bolt M10x60 | 850 | 1 |
| 5 | Steel adapter | 669 | 1 |
| 6 | Washer M12 | 803 | 1 |
| 7 | Bolt M12x80x1,25 | 891 | 1 |
| 8 | Aluminium cylinder | 1123 | 1 |
| 9 | 3M Tape | 3M | 2 |

Assembly to instruction

- Removal of lower fairing. On the imide is a screw clips.

- Make the first left or right side and after this, can make start opposite side.

- Dismantle original bolt from frame M12.

- Stick on the frame 3M tapes ID.9, insert aluminium cylinder ID.8 and mount steel adapter ID.5 from outside the frame.For gripping use bolt ID.7 and washer ID.6.

- This same make on opossite side.

- If you have mount both side you can moun fading on the bike.

- Into the protektor inser steel box ID.2.

- Under the protektor insert dural.reduction ID.3 and this set gripping on the steel adapter, use bolt ID.4

Note: When assembling, please use the glue on the threads.