H42SDM

Honda CB1100 2013-

Fitting kit left side:		     		       Fitting kit right side:

| ID | Description | Cat.n | QTY |
| --- | --- | --- | --- |
| 1 | Dural. adapter | 676 | 1 |
| 2 | Bolt M10x110 | 879 | 2 |
| 3 | Washer M10 | 804 | 3 |
| 4 | Nut M10 | 810 | 2 |
| 5 | Plastic´s cover | X4 | 2 |
| 6 | Dural.bracket for SLD | 537/B | 1 |
| 7 | Dural.cylinder d=70 | - | 1 |
| 8 | Bolt M12x100 | 986 | 1 |
| 9 | Bolt M6x20 | 1024 | 2 |
| 10 | Washer M6 | 1023 | 2 |
| 11 | Dural.cylinder d=14,5 | 1160 | 2 |

| ID | Description | Cat.n | QTY |
| --- | --- | --- | --- |
| 1 | Dural. adapter | 677 | 1 |
| 2 | Bolt M10x110 | 879 | 2 |
| 3 | Washer M10 | 804 | 3 |
| 4 | Nut M10 | 810 | 2 |
| 5 | Plastic´s cover | X4 | 2 |
| 6 | Dural.bracket for SLD | 537/B | 1 |
| 7 | Dural.cylinder d=50 | 208 | 1 |
| 8 | Bolt M12x80 | 956 | 1 |
| 9 | Bolt M6x20 | 1024 | 2 |
| 10 | Washer M6 | 1023 | 2 |

3

5

6

1

2

1

7

3

5

2

2

2

3

3

3

3

3

4

7

6

4

9,10

8

9,10

11

8

Instruction to assembly-left side:

- Take out orignal bolts from frame  and engine.

- Insert bolts ID.2 into the duralumin adapter ID.1. Under the adper insert dural.cylinders ID.11. This set mounting on frame. Thighten it by nuts ID.4 and washers ID.3.

- Take orignial bolt from engine M10, insert washer ID.3 on this bolt and make assebly.

- Put plastic covers into the bolts (see photo).

- Insert dural.bracket ID.6 together with dural.cylinder ID.7 on the bolt ID.8.

- This kit mounting on dural. adapter.

- For fixing slider use bolts with washers ID.10 and ID.9.

Instruction to assembly-right side:

- Take out orignal bolts from frame  and engine.

- Insert bolts ID.2 into the duralumin adapter ID.1. This set mounting on frame. Thighten it by nuts ID.4 and washers ID.3.

- Take orignial bolt from engine M10, insert washer ID.3 on this bolt and make assebly.

- Put plastic covers into the bolts (see photo).

- Insert dural.bracket ID.6 together with dural.cylinder ID.7 on the bolt ID.8.

- This kit mounting on dural. adapter.

- For fixing slider use bolts with washers ID.10 and ID.9.

Warning: We recommend use the glue of all threads. Medium strong.

Illustration photos, only for plac for gripping duralumin adapters.