H42

Honda CB1100 2013-

Fitting kit left side:		     		       Fitting kit right side:

| ID | Description | Cat.n | QTY |
| --- | --- | --- | --- |
| 1 | Dural. adapter | 676 | 1 |
| 2 | Bolt M10x110 | 879 | 2 |
| 3 | Washer M10 | 804 | 3 |
| 4 | Nut M10 | 810 | 2 |
| 5 | Plastic´s cover | X4 | 2 |
| 6 | Steel box | X2 | 1 |
| 7 | Dural.cylinder d=50 | 209 | 1 |
| 8 | Bolt M12x90 | 808 | 1 |
| 9 | Dural.cylinder d=14,5 | 1160 | 2 |

| ID | Description | Cat.n | QTY |
| --- | --- | --- | --- |
| 1 | Dural. adapter | 677 | 1 |
| 2 | Bolt M10x110 | 879 | 2 |
| 3 | Washer M10 | 804 | 3 |
| 4 | Nut M10 | 810 | 2 |
| 5 | Plastic´s cover | X4 | 2 |
| 6 | Steel box | X2 | 1 |
| 7 | Dural.cylinder d=30 | 170 | 1 |
| 8 | Bolt M12x70 | 977 | 1 |

3

5

6

1

6

1

7

3

5

2

2

3

3

3

3

4

7

4

8

9

8

2

2

Instruction to assembly-left side:

- Take out orignal bolts from frame  and engine.

- Insert bolts ID.2 into the duralumin adapter ID.1. Under the adper insert dural.cylinders ID.9. This set mounting on frame. Thighten it by nuts ID.4 and washers ID.3.

- Take orignial bolt from engine M10, insert washer ID.3 on this bolt and make assebly.

- Put plastic covers into the bolts (see photo).

- Insert steel box ID.6 into the protector with bolt ID.8.

- This kit mounting on dural. adapter.

Instruction to assembly-right side:

- Take out orignal bolts from frame  and engine.

- Insert bolts ID.2 into the duralumin adapter ID.1. This set mounting on frame. Thighten it by nuts ID.4 and washers ID.3.

- Take orignial bolt from engine M10, insert washer ID.3 on this bolt and make assebly.

- Put plastic covers into the bolts (see photo).

- Insert steel box ID.6 into the protector with bolt ID.8.

- This kit mounting on dural. adapter.

Warning: We recommend use the glue of all threads. Medium strong.