H43SD

Honda CBR650F 2014-

Left side mounting kit:		     	Right side mounting kit:

| ID | Descritpion | Cat.no | Pcs |
| --- | --- | --- | --- |
| 1 | Steel adapter | 674 | 1 |
| 2 | Dural. cylinder 20mm | 1152 | 1 |
| 3 | Dural. cylinder 28mm | 1151 | 1 |
| 4 | Bolt M10x65x1,25 | 945 | 1 |
| 5 | Washer M10 | 804 | 1 |
| 6 | Dural. adapter 20mm | 10 | 1 |
| 7 | Dural adapter for SLD | 537 | 1 |
| 8 | Bolt M10x60 | 850 | 1 |
| 9 | Bolt M8x25 | 815 | 2 |
| 10 | Washer M8 | 800 | 2 |

| ID | Description | Cat.no | Pcs |
| --- | --- | --- | --- |
| 1 | Steel adapter | 675 | 1 |
| 2 | Dural. cylinder 28 mm | 1151 | 1 |
| 3 | Bolt M10x65x1,25 | 945 | 1 |
| 4 | Washer M10 | 804 | 1 |
| 5 | Dural. adapter 20mm | 10 | 1 |
| 6 | Dural adapter for SLD | 537 | 1 |
| 7 | Bolt M12x50x1,25 | 990 | 1 |
| 8 | Bolt M10x40 | 854 | 1 |
| 9 | Bolt M8x25 | 815 | 2 |
| 10 | Washer M8 | 800 | 2 |

Left side instructions:

- Remove original bolt M10 and M12 from the frame of motorcycle.

- Fit steel adapter on the frame. Use original bolt and cylinder ID2 for front thread. Use Bolt ID4 and washer ID5 and cylinder ID3 for rear thread.

- Using bolt ID8 mount dural adapter ID6 and dural adapter for SLD ID7 on the steel adapter.

- Fit slider using two M8 bolts and washers.

Left side instructions:

- Remove original bolt M10 and M12 from the frame of motorcycle.

- Fit steel adapter on the frame. Use M12 bolt (ID7) for front thread. Use Bolt ID3 and  washer ID4 and cylinder ID2 for rear thread.

- Using bolt ID8 mount dural adapter ID5 and dural adapter for SLD ID6 on the steel adapter.

- Fit slider using two M8 bolts and washers.

We recommend to use the glue on all threads (medium strong).

9,10

7

2

9,10

4

5

7

3

1

4

3

2

5

8

6

1

8

6