H44SD

Honda CB650F 2014-

Left side mounting kit:		     	Right side mounting kit:

| ID | Description | Cat.no | Pcs |
| --- | --- | --- | --- |
| 1 | Steel adapter | 674 | 1 |
| 2 | Dural. cylinder 16mm | 1150 | 1 |
| 3 | Dural. cylinder 28mm | 1151 | 1 |
| 4 | Bolt M10x65x1,25 | 945 | 1 |
| 5 | Washer M10 | 804 | 1 |
| 6 | SLD adapter | 537 | 1 |
| 7 | Bolt M10x60 | 850 | 1 |
| 8 | Dural. cylinder 20mm | 10 | 1 |
| 9 | Bolt M8x25 | 815 | 2 |
| 10 | Washer M8 (spring) | 800 | 2 |

| ID | Description | Cat.no | Pcs |
| --- | --- | --- | --- |
| 1 | Steel adapter | 675 | 1 |
| 2 | Dural. cylinder 28mm | 1151 | 1 |
| 3 | Bolt M10x65x1,25 | 945 | 1 |
| 4 | Washer M10 | 804 | 1 |
| 5 | Bolt M10x40 | 854 | 1 |
| 6 | SLD adapter | 537 | 1 |
| 7 | Bolt M12x50x1,25 | 990 | 1 |
| 8 | Dural. cylinder 20mm | 10 | 1 |
| 9 | Bolt M8x25 | 815 | 2 |
| 10 | Washer M8 (spring) | 800 | 2 |

1

7

2

9

9

10

1

10

6

6

8

8

3

2

7

5

4

3

5

4

Left side instructions:

- Remove original bolts M12 and M10 from the frame of motorcycle.

- Mount steel adapter ID1 on the frame of motorcycle. Use cylinder ID2 and original bolt for tightening front part. Use cylinder ID3 and bolt ID4+ID5 for tightening rear part of adapter.

- Place SLD adapter ID6 and cylinder ID8 on bolt ID7 and mount this kit into the steel adapter ID1.

- Mount slider on SLD adapter using two bolts ID9 and washers ID10.

Right side instructions:

- Remove original bolts M12 and M10 from the frame of motorcycle.

- Mount steel adapter on the frame of motorcycle. Use bolt ID7 for tightening front part. Use bolt ID3+ID4 and cylinder ID2 for tightening rear part of adapter.

- Place SLD adapter ID6 and cylinder ID8 on bolt ID5 and mount this kit into the steel adapter ID1.

- Mount slider on SLD adapter using two bolts ID9 and washers ID10.

We recommend to use the glue on all threads (medium strong).