H44S

Honda CB650F 2014-

Fitting kit left side:		     		Fitting kit right side:

| ID | Description | Cat.n | Qt |
| --- | --- | --- | --- |
| 1 | Steel adapter | 674 | 1 |
| 2 | Dural. spacer d=16 | 1150 | 1 |
| 3 | Dural.spacer d=28 | 1151 | 1 |
| 4 | Bolt M10x65x1,25 | 945 | 1 |
| 5 | Washer M10 | 804 | 1 |
| 6 | Dural. adapter | 292 | 1 |
| 7 | Bolt M6x25 | 877 | 3 |
| 8 | Bolt M4x16 | 876 | 2 |
| 9 | Bolt M10x40 | 854 | 1 |

| ID | Description | Cat.n | Qt |
| --- | --- | --- | --- |
| 1 | Steel adapter | 675 | 1 |
| 2 | Dural.spacer d=28 | 1151 | 1 |
| 3 | Bolt M10x65x1,25 | 945 | 1 |
| 4 | Washer M10 | 804 | 1 |
| 5 | Dural. adapter | 292 | 1 |
| 6 | Bolt M12x50x1,25 | 990 | 1 |
| 7 | Bolt M6x25 | 877 | 3 |
| 8 | Bolt M4x16 | 876 | 2 |
| 9 | Bolt M10x30 | 882 | 1 |
| 10 |  |  |  |

6

2

1

5

6

1

9

9

8

8

7

5

3

7

2

4

4

3

Instruction to assembly left side:

- Take out original bolts from frame (arrows on the pictures).

- Insert dural. spacer ID.2 between the steel bracket and engine, front part of motorbike. For mounting use original bolt torque moment 55Nm.

- For rear gripping use dural. spacer ID.3. Spacer insert into the frame and put washer ID.5 on the bolt ID.4. Mounting steel bracket. Torque moment 40 Nm.

- Insert dural.adapter ID.6 on the bolt ID.9.

- This kit mounting on the steel adapter.

- The bolts ID.7 are for mounting slider. Into the protector insert alu. cover use bolts ID.8

Instruction to assembly right side:

- Take out original bolts from frame (arrows on the pictures).

- For mounting in front part use original bolt torque moment 55Nm ID.7.

- For rear gripping use dural. spacer ID.2. Spacer insert into the frame and put washer ID.4 on the bolt ID.3. Mounting steel bracket. Torque moment 40 Nm.

- Insert dural.adapter ID.5 on the bolt ID.9.

- This kit mounting on the steel adapter.

- The bolts ID.7 are for mounting slider. Into the protector insert alu. cover use bolts ID.8

Caution: When installing, we recommend using glue on the threads (blue color).