H44

Honda CB650F 2014-

Fitting kit left side:		     		Fitting kit right side:

| ID | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Steel bracket | 674 | 1 |
| 2 | Dural. spacer d=16 | 1150 | 1 |
| 3 | Dural.spacer d=28 | 1151 | 1 |
| 4 | Bolt M10x65x1,25 | 945 | 1 |
| 5 | Washer M10 | 804 | 1 |
| 6 | Steel box | X1 | 1 |
| 7 | Bolt M10x45 | 848 | 1 |

| ID | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Steel bracket | 675 | 1 |
| 2 | Dural.spacer d=28 | 1151 | 1 |
| 3 | Bolt M10x65x1,25 | 945 | 1 |
| 4 | Washer M10 | 804 | 1 |
| 5 | Steel box | X1 | 1 |
| 6 | Bolt M10x35 | 847 | 1 |
| 7 | Bolt M12x50x1,25 | 990 | 1 |

2

7

6

1

4

3

1

5,4

33

2

7

5

6

ID.7

Instruction to assembly left side:

- Take out original bolts from frame (arrows on the pictures).

- Insert dural. spacer ID.2 between the steel bracket and engine, front part of motorbike. For mounting use original bolt torque moment 55Nm.

- For rear gripping use dural. spacer ID.3. Spacer insert into the frame and put washer ID.5 on the bolt ID.4. Mounting steel bracket. Torque moment 40 Nm.

- Into the protector insert ID.6 and tighten protector by bolt ID.7

Instruction to assembly right side:

- Take out original bolts from frame (arrows on the pictures).

- For mounting in front part use original bolt torque moment 55Nm ID.7.

- For rear gripping use dural. spacer ID.2. Spacer insert into the frame and put washer ID.4 on the bolt ID.3. Mounting steel bracket. Torque moment 40 Nm.

- Into the protector insert ID.5 and tighten protector by bolt ID.6

Caution: When installing, we recommend using glue on the threads (blue color).