H45SDM

Honda CBR300R 2014-

Montážní kit levá strana:		     		Montážní kit pravá strana:

| ID | Description | Cat.no | pcs |
| --- | --- | --- | --- |
| 1 | Steel adapter | 684 | 1 |
| 2 | Dural. Cylinder - 5mm | 16 | 1 |
| 3 | Bolt M6x60 | 872 | 1 |
| 4 | Dural.cylinder - 25mm | 1159 | 1 |
| 5 | Nut M10 | 810 | 1 |
| 6 | Washer M6 | 947 | 1 |
| 7 | Alu. adapter | 537/B | 1 |
| 8 | Alu. Cylinder - 70mm | 15 | 1 |
| 9 | Bolt M10x90 | 880 | 1 |
| 10 | Bolt M6x20 | 1024 | 2 |
| 11 | Washer M6 | 1023 | 2 |
| 12 | M10x300x30x30 | - | 1 |

| ID | Description | Cat.no | pcs |
| --- | --- | --- | --- |
| 1 | Steel adapter | 685 | 1 |
| 2 | Dural. Cylinder - 5mm | 16 | 1 |
| 3 | Bolt M6x60 | 872 | 1 |
| 4 | Dural.cylinder - 25mm | 1159 | 1 |
| 5 | Nut M10 | 810 | 1 |
| 6 | Washer M6 | 947 | 1 |
| 7 | Alu. adapter | 537/B | 1 |
| 8 | Alu. Cylinder - 70mm | 15 | 1 |
| 9 | Bolt M10x90 | 880 | 1 |
| 10 | Bolt M6x20 | 1024 | 2 |
| 11 | Washer M6 | 1023 | 2 |

Instructions:

- Remove bottom fairing of the left side of motorcycle.

- Remove original bolt from frame of motorcycle on the left side.

- Remove bolt M6 from the engine bonnet.

- Insert steel rod (ID12) trough the frame and engine. Put cylinder (ID2) and steel adapter (ID1) on it. Tighten adapter using nut (ID5) and original bolt.

- Insert cylinder (ID4) between steel adapter and engine bonnet.

- Now tighten all bolts and put fairing back.

- Insert bolt (ID9) into alu adapter (ID7) and alu cylinder (ID8).

- Fit this set on the steel adapter (ID1)

- In the end fit slider on the alu adapter using two bolts and washers (ID10 and ID11).

- Repeat same process on the right side.

We recommend to use the glue on all threads (blue colour – medium strong).

2

1

4

3

1

3

2

4

5

5

7

7

12

8

8

9

9

10,11

10,11