H50S

Honda MSX 125_2014-16

Fitting kit left side:		     		  Fitting kit right side:

| ID | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1.1 | Steel adapter | 690 | 1 |
| 2.1 | Alu. spacer d=57mm | 1166 | 1 |
| 3.1 | Stainless steel rod 10x225x30x30 |  | 1 |
| 4 | Nut M10 | 810 | 2 |
| 5 | Washer M10 | 804 | 2 |
| 6.1 | Bolt M6x40 | 865 | 2 |
| 7 | Alu.spacer l=10mm | 1164 | 1 |
| 8 | Alu.spacer l=9mm | 1165 | 1 |

| ID | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1.2 | Steel adapter | 691 | 1 |
| 2.2 | Alu. spacer d=21mm | 1158 | 1 |
| 3.2 | Connecting bar 12x230xM10x20x20 |  | 1 |
| 4 | Nut M10 | 810 | 2 |
| 5 | Washer M10 | 804 | 2 |
| 6.2 | Bolt M6x50 | 972 | 1 |

picture 1

Kit for protectors:

| ID | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 9 | Plastic cap | X35 | 4 |
| 10 | Alu. spacer d=70 (right) | 7 | 1 |
| 11 | Alu. spacer d=50 (left) | 5 | 1 |
| 12 | Bolt M10x100 (right) | 885 | 1 |
| 13 | Bolt M10x80 (left) | 852 | 1 |
| 14 | Bolt M6x25 | 877 | 6 |
| 15 | Bolt M4x16 | 876 | 4 |
| 16 | Nut M10 | 810 | 2 |
| 17 | Slider adapter | 313 | 2 |

Slider adapter

cylinder

Steel adapter

Nut

Bolt

picture 2

15

9

13+17+11+16

14

12+17+10+16

2.1

2.2

1.2

1.1

6.1+8

6.1+10

6.2

Instructions for fitting kit:

- Dismantle nut on the left side of engine.

- Remove original bolts from engine.

- Fit mounting kit according to the picture 2.

  - Shorter steel rod (ID3.1) pass through engine.

  - Shorter alu spacer (ID8) is on the lower bolt M6 (ID6.1).

  - Longer alu spacer (ID2.1) is on the left

- Cover all four M10 bolts using plastic caps (ID9)

Instructions for sliders:

- Fit sliders accoridnig to the picture 1.

  - Put slider adapter (ID17), alu spacer (ID10 on the right, ID11 on the left) on M10 bolt (ID12 on the right, ID13 on the left).

  - Tighten the M10 bolts using M10 nuts (ID16).

  - Fit slider on the slider adapter (ID17) using three M6 bolts (ID14).

  - Fit coloured cover using two M4 bolts (ID15).

We recommended to use the glue on all threads (medium strong – blue colour).

6

6

9

9

10

12

16

17.18