H50

Honda MSX 125

Fitting kit left side:		     		  Fitting kit right side:

| ID | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Fitting bracket | 690 | 1 |
| 2 | Alu. spacer d=57mm | 1166 | 1 |
| 3 | Stainless steel rod 10x225x30x30 |  | 1 |
| 4 | Nut M10 | 810 | 2 |
| 5 | Washer M10 | 804 | 2 |
| 6 | Bolt M6x40 | 865 | 2 |
| 7 | Alu.spacer l=10mm | 1164 | 1 |
| 8 | Alu.spacer l=9mm | 1165 | 1 |

| ID | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Fitting bracket | 691 | 1 |
| 2 | Alu. spacer d=21mm | 1158 | 1 |
| 3 | Connecting bar 12x230xM10x20x20 |  | 1 |
| 4 | Nut M10 | 810 | 2 |
| 5 | Washer M10 | 804 | 2 |
| 6 | Bolt M6x50 | 972 | 1 |

Kit for protectors:

| ID | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 9 | Plastic cap | X35 | 4 |
| 10 | Alu. spacer d=70 | 7 | 1 |
| 11 | Alu. spacer d=50 | 5 | 1 |
| 12 | Bolt M10x95 | 822 | 1 |
| 13 | Bolt M10x110 | 879 | 1 |
| 14 | Steel box | X1 | 2 |
| 15 | Washer | 801 | 2 |
| 16 | Nut M10 | 810 | 2 |

Fitting instruction:

- Dismantle nut on the left side of engine. As shown on the picture.

- Take off the bolt from engine.

- From left side take off two bolts from engine case.

- Shorter steel rod d=225mm ID.3 place through the engine, replace orignal bolt.

- Insert spacer d=57mm ID.2 from lef side. Insert spacer d=21mm ID.2 from right side. Now place steel brackets and easy tighten nut ID.4 with washer  ID.5.

- Assembly bolts for engine case ID.6 for right side.

- Use bolts ID.6 from left side. Insert alu spacer d=9mm on the bolt M6 from back side.

- Place steel rod infront part, for tighten use nut ID.4 and washers ID.5.

- Now tighten all connections and bolts, place plastic covers on the nuts.

Assembly for cash protectors:

- Left side:

- On the bolt ID.12 insert steel box ID.14. This kit insert inside the protector. Under the protector place alu. spacer ID.11.

- This kit assembly on the steel bracke, tighten it by nut ID.15 and washer ID.16 from back side.

- Right side:

- On the bolt ID.13 insert steel box ID.14. This kit insert into the protector. Place alu. spacer under the protector ID.10.

- This kit assembly on the steel bracket and tighte it by nut ID.15 with washer ID.16 from the back side.

During assembly use glue of all threads. Medium strong.

6

6

9

9

10

12

16

17.18

2

6

7

2

1

4,5

9

3

4,5

15,16

4,5

8

14

12

11

4,5

9

1

3

2

9

9

10

14

13

15,16

6