H53S

Honda CBR500R 2016-

Left side kit:		     		       		Right side kit:

| ID | Desription | Cat.no | Pcs |
| --- | --- | --- | --- |
| 1 | Bolt M10x80x1,25 | 920 | 1 |
| 2 | Washer M10 | 804 | 1 |
| 3 | Bolt M6x35 | 989 | 2 |
| 4 | Dural. cylinder - 30,5mm | 197 | 1 |
| 5 | Slider adapter | 317 | 1 |
| 6 | Bolt M10x80 | 852 | 1 |
| 7 | Steel adapter | 714 | 1 |
| 8 | Special cylinder | 190 | 1 |
| 9 | Bolt M6x25 | 877 | 3 |
| 10 | Bolt M4x16 | 876 | 2 |

| ID | Desription | Cat.no | Pcs |
| --- | --- | --- | --- |
| 1 | Bolt M10x90x1,25 | 890 | 1 |
| 2 | Washer M10 | 804 | 1 |
| 3 | Bolt M6x35 | 989 | 2 |
| 4 | Dural. cylinder - 44mm | 199 | 1 |
| 5 | Slider adapter | 317 | 1 |
| 6 | Bolt M10x70 | 851 | 1 |
| 7 | Ocelový adaptér | 715 | 1 |
| 8 | Special cylinder | 191 | 1 |
| 9 | Bolt M6x25 | 877 | 3 |
| 10 | Bolt M4x16 | 876 | 2 |

2

4

3

Instructions:

- Remove fairing of motorcycle.

- Remove original bolt from the frame.

- Mount steel adapter ID7 using two bolts ID3 and bolt ID1+ID2+ID4.

- Mount fairing back on the motorcycle.

- Insert bolt ID6 into adapter ID5.

- Put special cylinder ID8 on the bolt behind adapter.

- Mount this kit into the steel adapter.

- Mount slider using three bolts M6 (ID9).

- Mount cover using two bolts M4 (ID10).

We recommend to use the glue on all threads (medium strong).

ID8

ID1+ID2+ID4

ID7

ID3

ID3

ID5

ID6