| H55 ID | Description | Cat. No. | Pcs. |
| --- | --- | --- | --- |
| 1 | Steel insert M10 | X1 | 1 |
| 2 | Bolt M12x60x1,25mm | 916 | 1 |
| 3 | Bolt M10x55mm | 1002 | 1 |
| 4 | Steel adapter H55-P | 727 | 1 |
| 5 | Tape 3M |  | 1 |
| 6 | Cover plate – P | 729 | 1 |
| 7 | Bolt M5x6 | 1046 | 1 |
| 8 | Plastic panel –P | 731 | 1 |

Honda CBR 1000rr ´2017-

|  | Kit left side: |  |  |  |  | Kit right side: |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| ID | Description | Cat. No. | Pcs. |
| --- | --- | --- | --- |
| 1 | Steel insert M10 | X1 | 1 |
| 2 | Bolt M12x55x1,25mm | 975 | 1 |
| 3 | Bolt M10x55mm | 1002 | 1 |
| 4 | Steel adapter H55-L | 726 | 1 |
| 5 | Tape 3M | - | 1 |
| 6 | Cover plate – L | 728 | 1 |
| 7 | Bolt M5x6 | 1046 | 1 |
| 8 | Plastic panel –L | 730 | 1 |

Mounting instructions – left side:

- Remove the original plastic from side and M12 bolt from the motorcycle frame.

- Attach the 3M tape (ID.5) to the point where motorcycle frame meets ID.2 blt and tighten the steel adapter towards the motorcycle frame.

- Replace the original black plastic with plastic panel (ID.8) at the rear hood.

- Check that everything is in place and proceed to return the side plastic to its place.

- Use the ID.7 bolt to fit the cover plate (ID.6) onto the adapter.

- Push in the steel insert (ID.1) together with ID.3 bolt.

- Mount the full assembly into the adapter thread.

Mounting instructions – right side:

- Remove the original side plastic and M12 bolt from the motorcycle frame

- Attach the 3M tape (ID.5) to the point where motorcycle frame meets ID.2 blt and tighten the steel adapter towards the motorcycle frame.

- Replace the original black plastic with plastic panel (ID.8) at the rear hood.

- Check that everything is in place and proceed to return the side plastic to its place.

- Use the ID.7 bolt to fit the cover plate (ID.6) onto the adapter.

- Push the steel insert (ID.1) into the protector together with ID.3 bolt.

- Mount the full assembly into the adapter thread.

Note  - we recommend using medium strenght glue (blue color) for threads of all screws.