H5SDM

Honda CBF600 2008-

Honda CB600F Hornet 2007-

Kit for left side:					Kit for right side:

| ID | Description | Cat.No | Qty |
| --- | --- | --- | --- |
| 1 | Slider SLDM | SLDM | 1 |
| 2 | Bolt M6x20 | 1024 | 2 |
| 3 | Washer M6 | 1023 | 2 |
| 4 | Bolt M10x40 | 854 | 1 |
| 5 | Dural. adapter for SLD | DV-10 | 1 |
| 6 | Dural. adapter | 609 | 1 |
| 7 | Bolt M12x85x1,25mm | 899 | 1 |
| 8 | Bolt M10x65x1,25mm | 945 | 1 |
| 9 | dural.cylinder d=12mm | 80 | 1 |
| 10 | dural.cylinder d=8mm | 79 | 1 |
| 11 | Washer M10-spring | 801 | 1 |
| 12 | Dural.bracket for SLDM | 537/B | 1 |

| ID | Description | Cat.No | Qty |
| --- | --- | --- | --- |
| 1 | Slider SLDM | SLDM | 1 |
| 2 | Bolt M6x20 | 1024 | 2 |
| 3 | Washer M6 | 1023 | 2 |
| 4 | Bolt M10x40 | 854 | 1 |
| 5 | Dural. adapter for SLD | DV-10 | 1 |
| 6 | Dural. adapter | 610 | 1 |
| 7 | Bolt M12x65x1,25mm | 940 | 1 |
| 8 | Bolt M10x65x1,25mm | 945 | 1 |
| 9 | dural.cylinder d=12mm | 80 | 1 |
| 10 | Washer M10-spring | 801 | 1 |
| 11 | Dural.bracket for SLDM | 537/B | 1 |

Instruction to assembly-left side:

- Take out orignals bolt from frame.

- Insert bolts ID.7 and ID.8 into the dural.adapter ID.6.

- Put on the bolts dural.cylinder ID.9 and ID.10, and mount this set on the frame, use the original bolt and washer.

- Take bolt ID.4 put on the bolt washer M10 ID.11, dural.bracket for SLD ID.12 and dural.adapter for SLD ID.5

- This kit mount on the frame.

- Use bolts and washer ID.2 a ID.3 and mount DIAMOND.

Instruction to assembly-right right side:

- Take out orignals bolt from frame.

- Insert bolts ID.7 and ID.8 into the dural.adapter ID.6.

- Put on the bolts dural.cylinder ID.9 from back side the dural. holder.

- Mount this kit on the frame.

- Take bolt ID.4 put on the bolt washer M10 ID.10, dural.bracket for SLD ID.11 and dural.adapter for SLD ID.5

- This kit mount on the frame.

Use bolts and washer ID.2 a ID.3 and mount DIAMOND

Warning – use the glue of all threads. Medium strong.