H5S

Honda CBF600 2008-

Honda CB600F Hornet 2007-

Kit for left side:					Kit for right side:

| ID | Popis | Kat.č. | Počet |
| --- | --- | --- | --- |
| 1 | Slider SL01 | SL01 | 1 |
| 2 | Bolt M6x25 | 877 | 3 |
| 3 | Bolt M4x16 | 876 | 2 |
| 4 | Bolt M10x20 | 819 | 1 |
| 5 | Adapter for slider | DV 317 | 1 |
| 6 | Dural. adaptér | 609 | 1 |
| 7 | Bolt M12x85x1,25mm | 899 | 1 |
| 8 | Bolt M10x65x1,25mm | 945 | 1 |
| 9 | dural.cylinder d=12mm | 80 | 1 |
| 10 | dural.cylinder d=8mm | 79 | 1 |
| 11 | Cover | CapSl. | 1 |

| ID | Popis | Kat.č. | Počet |
| --- | --- | --- | --- |
| 1 | Slider SL01 | SL01 | 1 |
| 2 | Bolt M6x25 | 877 | 3 |
| 3 | Bolt M4x16 | 876 | 2 |
| 4 | Bolt M10x20 | 819 | 1 |
| 5 | Adapter for slider | DV 317 | 1 |
| 6 | Dural. cylinder | 610 | 1 |
| 7 | Bolt M12x65x1,25mm | 940 | 1 |
| 8 | Bolt M10x65x1,25mm | 945 | 1 |
| 9 | dural.cylinder d=12mm | 80 | 1 |
| 10 | Cover | CapSl. | 1 |

2

1

3

2

3

1

6

6

5

5

8

8

11

9

7

9

10

7

Instruction- left side :

- Take out bolts from frame

- Dural. adapter ID.6 (smaller) mounting on the frame, use bolts ID.7 and ID.8

- Insert between the adapter and frame dural. cylinder ID.9 and ID.10

- Mount adapter for slider ID.5 use bolt M10x20 ID.4

- Mout slider use bolts M6x25 ID.2

- Insert cap into the slider ID.11 and use bolts M4x16 ID.3

Instruction- right side :

- Take out bolts from frame

- Dural. adapter ID.6 mounting on the frame, use bolts ID.7 and ID.8

- Insert between the adapter and frame dural. cylinder ID.9

- Mount adapter for slider ID.5 use bolt M10x20 ID.4

- Mout slider use bolts M6x25 ID.2

- Insert cap into the slider ID.11 and use bolts M4x16 ID.3

Warning - the threads of the screws is recommended to use premium glue moderate (blue color).