H6S

Honda CBF1000 2006-2010

| Left side kit |  |  |  |  | Right side kit |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.no | Pcs |  |  | ID | Description | Cat.no. | Pcs |  |
| 1 | Dural. adapter | DV-314 | 1 |  |  | 1 | Dural. adapter | DV-314 | 1 |  |
| 2 | Bolt M6x25 | 877 | 3 |  |  | 2 | Bolt M6x25 | 877 | 3 |  |
| 3 | Bolt M4x16 | 876 | 2 |  |  | 3 | Bolt M4x16 | 876 | 2 |  |
| 4 | Bolt M12x90x1,25mm | 931 | 1 |  |  | 4 | Bolt M12x80x1,25mm | 908 | 1 |  |

Instructions:

- Remove original bolt.

- Mount dural. adapter ID1 on the frame of motorcycle using bolt ID4.

- Fit slider on the slider adapter using three bolts ID2.

- Insert cover into slider and tighten it using two bolts ID3.

We recommend to use the glue (medium strong).