K11S

Kawasaki Z750 2007-2012

Fitting kit left side:		     		Fitting kit right side:

| ID | Description | Cat.N | Qty |
| --- | --- | --- | --- |
| 1 | Dural.adapter SL01 | 313 | 1 |
| 2 | Bolt M10x30mm | 882 | 1 |
| 3 | Bolt M6x25 | 877 | 3 |
| 4 | Bolt M4x16 | 876 | 2 |
| 5 | Bolt M8x60mm | 838 | 2 |
| 6 | Dural.cylinder d=14mm | 18 | 2 |
| 7 | Dural.adapter | 547 | 1 |
| 8 | Washer spring M10 | 801 | 1 |
| 9 | Plastic cap | X5 | 2 |
| 10 | 3M tape |  | 1 |

| ID | Description | Cat.N | Qty |
| --- | --- | --- | --- |
| 1 | Dural.adapter SL01 | 292 | 1 |
| 2 | Bolt M10x35mm | 847 | 1 |
| 3 | Bolt M6x25 | 877 | 3 |
| 4 | Bolt M4x16 | 876 | 2 |
| 5 | Bolt M8x60mm | 838 | 2 |
| 6 | Dural.cylinder d=14mm | 18 | 2 |
| 7 | Dural.adapter | 547 | 1 |
| 8 | Plastic cap | X5 | 2 |
| 9 | 3M tape |  | 1 |
| 10 | Washeer spring M10 | 801 | 1 |

8

10

2

7

7

2

6

1

1

5

5

6

8

4

3

4

3

9

10

9

Instruction to assembly left side:

- Take out original bolts from frame M8.

- Mounting dural.adapter ID.7 on frame. Use bolts ID.5 and into the frame put dural.cylinders ID.6.

- You can stick 3M tape under the dural. adapter ID.7.

- Put plastic cover into the bolts ID.9.

- Grip dural.adapter ID1 on the bike. Use bolt ID.2 and washer ID.8

- Mounted slider on the adapter, use bolts ID.3.

- Insert cover into the slider, use bolts ID.4.

Instruction to assembly right side:

- Take out original bolts from frame M8.

- Mounting dural.adapter ID.7 on frame. Use bolts ID.5 and into the frame put dural.cylinders ID.6.

- You can stick 3M tape under the dural. adapter ID.6.

- Put plastic cover into the bolts ID.8.

- Grip dural.adapter ID.1 on the bike Use bolt ID.2 and washer ID.10.

- Mounted slider on the adapter, use bolts ID.3.

- Insert cover into the slider, use bolts ID.4.

Warning: We recommend use the glue of all threads. Medium strong.

Foto staršího typu uchycení slideru.